### Application of TOPSAR and Polarimetric SAR

### for Geological Mapping within north western Tasmania

### Dr Rob Hewson

### RMIT University

May 2015

# Table of Contents

1 Introduction											6

1.1 Polarimetric radar and TOPSAR basics						6

1.2  Objectives									9

2 Datasets, processing and quality control issues						10

2.1  TOPSAR Polarimetric 								10

2.2  TOPSAR DEM									11

2.3  Issues affecting radar interpretation and quality control 			12

2.3.1	 Incident angle 							12

2.3.2	 Surface roughness							14

2.2.3	 Dielectric properties and moisture					15

3 Results 											17

3.1 TOPSAR DEM 									17

3.1.1    Image products							17

3.1.2    Comparison with SRTM 25, Photogrammetric derived and

LiDar DEMs								20

3.2 TOPSAR multi-wavelength and multi-polarimetric products			24

3.2.1    TS1340 Northern study area 						25

3.2.2    TS1340 Southern study area 					28

3.2.3    Mosaicked polarimetric imagery					33

4 Conclusions											36

5 Acknowledgements										36

6 References											37

7 Appendix 1 TOPSAR Registration coordinate information.				39

Appendix 2 Statistical comparison between DEM results from TOPSAR acquisitions     43

Appendix 3 Statistical comparison between DEM results from TOPSAR (TS1297), 		          SRTM25m, Photogrammetric 25m and LiDar (Meredith)			46

# Table of Figures

Figure 1 Remote sensing regions including radar in relation to the

electromagnetic spectrum. 									6

Figure 2 Operation of a radar system.							7

Figure 3 Volume and surface scattering modes for transmitted and

returned radar.  										8

Figure 4 a) Schematic diagram of TOPSAR operation, and b) its

corresponding geometry.									9

Figure 5 Location of 1993 POLSAR (AIRSAR) and 2000 PACRIM

TOPSAR within north western Tasmania.							10

Figure 6 Schematic diagram illustrating the interaction between topography

and the radar depression angle on the resulting radar image effects of

layover and shadowing (Elachi & Van Zyl, 2006).						13

Figure 7 a) Relationship between look angle, depression angle and incident

angle assuming a relatively flat ground surface (Ford, 1998);

b) Schematic diagram illustrating local incident angle θloc and the

average incident angle θi resulting from local slope angle αloc (Raney, 1998).		13

Figure 8  Penetration depths for soil with different moisture contents at

different wavelengths e.g. C band (λ ~ 3cm) and L band (λ ~ 23cm),

after Dobson (1993).										16

Figure 9 a) TOPSAR DEM mosaic of TS1297, TS1339 and TS1340

acquisitions displayed with a linear 100 to 900 metre elevation stretch;

b) Shaded mosaic illuminated with 45o NE azimuth and 60 o  elevation;

and c) Slope mosaic product displayed with a linear 0 to 30 degrees slope stretch.	18

Figure 10 Supplied calculated local incident angle estimates for flight line TS1340.	19

Figure 11 Depiction of the three TOPSAR acquisitions and their overlay areas. 		19

The spatial extents are the same as shown in Figure 9 a-c).

Figure 12 Areas of coincident TOPSAR (blue:low; red:high), SRTM 25m,

Photogrammetric 25m and LiDar DEMs overlaying 1:250,000 geology.

The Meredith Granite (DG, Dga - blue outline) and non Meredith granite (black outline) highlight the two training areas for statistical comparison.					22

Figure 13 AIRSAR/TOPSAR Cvv image of flight line TS1340 showing the

locations of the vegetation (white box) and geological (magenta box) study areas.		24

Figure 14 a) Published 1:250000 geology of the vegetation study area;

b) 1:100000 topographic map; c)  shaded SRTM 25m DEM with

1:250000 geology overlay; d) Pseudocoloured Cvv backscatter response

(blue:low backscatter return; red high backscatter) with

1:25000 vegetation overlay. 									26

Figure 15 Pseudocoloured Lhv backscatter response

(blue:low backscatter return; red high backscatter) with 1:25000 vegetation overlay. 	27

Figure 16 Artifically shaded TOPSAR DEM (45 deg NE azimuth,

60 deg elevation)  with 1:250000 published geology. 					28

Figure 17 a) 1:250000 topographic map of TS1340 southern study

area as shown in Figure 11; b) 1:2500000 published geology with

Tertiary basalt highlighted in yellow; c) Radiometric Ternary image

(RGB : KThU) with geology overlay.; d) High frequency Tilt filtered aeromagnetics. 	29

Figure 18 a)  Shaded SRTM 25m DEM (45 deg NE azimuth, 60 deg elevation) with 1:250000 geology overlay. Tertiary basalt highlighted within red hatched area.

b) Shaded TOPSAR DEM (45 deg NE azimuth, 60 deg elevation)

with 1:250000 published geology overlay. 							30

Figure 19 Pseudocoloured Phv backscatter response

(blue:low backscatter return; red high backscatter) with

1:250000 geology overlay.									31

Figure 20 Pseudo coloured Phv backscatter response

(blue:low backscatter return; red high backscatter) with 1:25000 vegetation overlay. 	32

Figure 21 RGB composite of Phh, Pvv, Phv backscatter response

with 1:250000 geology overlay. 								32

Figure 22 a) RGB composite of Cvv, Ltp and Ptp with uniform linear stretch applied

to each band of combined TOPSAR acquisitions; b)  RGB composite of Cvv, Ltp

and Ptp with 99.0% linear stretch applied independantly to each band of TS1297,

TS1339 and TS1340. 										34

Figure 23 a) Greyscale imagery of Cvv with 99.0% linear stretch applied

independantly to each band of TS1297, TS1339 and TS1340;

b)  Greyscale imagery of Phv with uniform linear stretch applied to

combined TOPSAR Phv.									35

Figure A1 Shaded mosaicked TOPSAR DEM with location of

ground control points used for registration of each of the three

TOPSAR flight lines, TS 1297, 1339 and 1340.						39

# Table of Tables

Table 1 Definitions of smooth and rough radar surfaces for

AIRSAR/TOPSAR radar band wavelengths and maximum-minimum

range of look angles where Look angle = Incident angle,

assuming a flat ground surface.								15

Table 2 Statistical comparison between coincident TOPSAR,

SRTM 25m and Photogrammetry derived DEMs.						20

Table 3 Statistical comparison between coincident TOPSAR,

SRTM 25m, Photogrammetry  and LiDar derived DEMs.					21

Table 4 Spot comparisons between the TOPSAR, SRTM 25m, Photogrammetric 25m

and Lidar DEMs with five 1:25,000 topographic located hilltops.				23

# 1 Introduction

## 1.1 Polarimetric radar and TOPSAR basics

Radar remote sensing generates imagery that characterises some of the physical properties (morphology, roughness, dielectric properties  and geometric shapes) of the ground surface, including its cover (e.g. vegetation) and near-surface volume (e.g. rock outcrops and regolith layers) (Raney, 1998). It differs from optical or thermal remote sensing in its use of active microwave electromagnetic signals (Figure 1) and interpretation of their backscattered return for electric field strength and polarisation. The operation of radar from either a satellite or airborne platform involves a side looking pulse of microwave radiation transmitted perpendicular to the flight (or orbit) “range’ direction and observed by a receiver antenna (Figure 2). A variation of radar systems known as Synthetic Aperture Radar (SAR) optimises the spatial resolution using the Doppler return of the signal along the azimuth flight path of the platform.

Figure 1 Remote sensing regions including radar in relation to the electromagnetic spectrum. The relative wavelengths of the active radar bands K, X, C and P are shown. Note L band has a wavelength between C and P bands.

Figure 2 Operation of a radar system

The application radar for the extraction of geological information relies on the surface geometry and its contrasting dielectric properties, typically related to moisture content  or mineral composition (Ford, 1998). The geological interpretation of image enhancements are particularly suited to landform analysis from which geomorphological and geological structural interpretation can be made (Tapley, 2002). The radar technique provides its own illumination enabling observations to be independent of cloud cover, light rain, smoke haze and solar illumination, thus allowing all-time observation through all seasons and in all climatic regions. An important capability of radar is the ability to select the illumination geometry,  that  is, the incidence and azimuth angles, to highlight geological structure and other diagnostic properties of the terrain. Case studies describing SAR radar surveys within densely forested areas of Indonesia and Papua New Guinea have still shown its ability to map both structural features and erosional characteristics of the underlying rocks (Ford, 1998).

Radar technologies such as NASA’s multi wavelength polarimetric AIRSAR systems (Lou, 2002; http://airsar.jpl.nasa.gov/) include the include the longer wavelength P (68 cm) and L (25 cm) bands. AIRSAR has the potential capability to measure backscattered signal returns beneath forest canopies related to surface rock outcrops and sub surface regolith (Ford - Henderson and Lewis, 1998; Evans et al., 1988). The AIRSAR system incorporates multi-polarimetric combinations of transmitted horizontal (H) or vertical (V) electric field signals, and the detection of returning horizontal (H) or vertical (V) electric field returned backscatter. This facilitates the modelling of surface and volume radar scattering associated with vegetation and/or regolith (Figure 3). For example, surface scattering is more likely associated with like-polarized transmitted and returned signals (e.g. HH or VV) while volume scattering is associated with cross-polarized transmitted and returned signals (e.g. HV) (Figure 3).  A complete multi -wavelength -polarimetric AIRSAR data set provides all these combinations for C, L and P bands including a total power backscatter measurement (TP) Note that a HV return is assumed to be equivalent to VH polarisation.

E.g. C-HH, C-VV, C-HV, C-TP, L-HH, L-VV, L-HV, L-TP, P-HH, P-VV, P-HV, P-TP

Generally the bands associated with longer AIRSAR radar wavelengths, P and to a lesser extent L, measure more backscatter from beneath the vegetation canopy and ground surface and or regolith (Tapley, 2002). In particular, the HV polarization for P (P-HV) band provides the best indication of volume scattering from the shallow subsurface associated with the rock outcrops and regolith. In forested environments, HH polarisation signals potentially suffer less attenuation  from  the vertically aligned tree trunks and are more likely to provide information about the physical characteristics of the underlying ground-surface (Tapley, 2002). The cross-polarized  HV scattering  bands are less dependent on incidence angle, varying perpendicular across the flight line,  than the co-polarized (HH and VV) backscatter (Tapley, 2002).  Consequently HV returns are also less sensitive to variations in terrain slope. Geobotanical relationships, between geological units and vegetation species or forest type are potentially highlighted by the  VV polarisation data which is associated with increased interaction  with the vegetation and tree trunks (Tapley, 2002).

Figure 3 Volume and surface scattering modes for transmitted and returned radar.

In addition, AIRSAR’s C band (5.7 cm) is utilised within its TOPSAR system for interferometric topographic mapping (Zebker et al., 1992; Madsen et al, 1993) (Figure 4 a & b). Observing the returned radar backscatter of the C band signal with two detectors, A1 and A2 (Figure 4a) enables the determination of height, h, for each scanned pixel, z(x) (Figure 4b). As TOPSAR uses the C-VV wavelength and polarisation, its returned backscatter signals  are typically biased by the vegetation canopy.

Figure 4 a) Schematic diagram of TOPSAR operation, and b) its corresponding geometry.

## 1.2  Objectives

The aim of this report is to describe the processing and preliminary geological interpretation of TOPSAR acquired by Mineral Resources Tasmania from three flight lines in north western Tasmania during the PACRIM 2 Campaign, August 2000 (Figure 5). As part of this study, the quality control issues of the resulting Digital Elevation Model (DEM) and its comparisons with supplied LiDAR, SRTM 25 metre and 1:25000 Photogrammetric DEM products are  examined. The fully polarimetric AIRSAR/POLSAR Mt Read data, acquired in 1993 (Figure 5), was not able to be processed with the current ENVITM software (http://www.exelisvis.com/ProductsServices/ENVIProducts/ENVI.aspx). Its data format appears to be in an atypical format and its processing and interpretation awaits an update by the suppliers of  ENVITM .

Figure 5 Location of 1993 POLSAR (AIRSAR) and 2000 PACRIM TOPSAR within north western Tasmania.

# 2 Datasets, processing and quality control issues

## 2.1  TOPSAR Polarimetric

The PACRIM 2 TOPSAR data comprised nine channels comprising C-VV, L-HH, L-VV, L-HV, L-TP, P-HH, P-VV, P-HV, and P-TP. Three flight lines of TOPSAR imagery, TS1297, TS1340 and TS1339 were acquired of approximately 60 x 10 km, 60 x 10 km and 30 x 10 km image areas respectively (Figure 5). These were imported into ENVITM  software which facilitates the synthesis of the multiple wavelengths and polarisations from the Stokes Matrix Compression format (http://airsar.jpl.nasa.gov/documents/dataformat.htm). The nine channel imagery was then registered in a three stage process :

1) polarimetric TOPSAR imagery were registered using the WGS 84 geographic coordinates within the supplied header information for the corners (Appendix 1);

2) the WGS 84 projected imagery was converted to GDA94 MGA Zone 55S Easting and Northing coordinate projection;

3) The GDA94 MGA Zone 55 was registered using 20 to 30 ground control points identified by features (watercourse/road intersections, dam walls, mountain peaks etc) identified on supplied MRT 1:25000 topographic maps, SPOT imagery and 1:25,000, and 1:25000 Photogrammetric and SRTM 25 metre DEMs (Figure A1, Appendix 1).

The resulting GDA94 MGA Zone 55 polarimetric TOPSAR imagery was imported into ERDAS ERMapperTM   (http://erdas-er-mapper.software.informer.com/) for integration and comparison with supplied MRT datasets. Some issues of systematic spatial noise were apparent and likely to have resulted from AIRSAR system and aircraft electronics (Tapley, 2002). Two dimensional (2D) Fast Fourier Transform  (FFT) 2D band cut  filtering was attempted using ENVITM however difficulty was encountered in the exclusion of the systematic spatial frequency noise while not introducing noise from the user defined band cuts. As a consequence, no FFT filtering was applied to this TOPSAR imagery. However a more serious noise issue was apparent from radar “speckle”, associated with inherent random and multiplicative returning coherent backscatter (Tapley, 2002). Various adaptive filtering techniques using both ENVITM and ERDAS ERMapperTM software including Median, Gamma, Froist and Lee filters (Lopes et al., 1990). The standard Lee filter (Lee, 1980) available using ERDAS ERMapperTM  was found to give the best result in reducing the speckle for this AIRSAR/TOPSAR imagery. This operation of this filter is based on the probability of a Gaussian distribution where 95.5% of random samples are assumed to be within a 2 standard deviation (2 sigma) range. This noise suppression filter replaces the pixel of interest with the average of all DN values within a 5 x 5 pixel sample moving window that fall within the designated range (ERDAS, 1999)

## 2.2  TOPSAR DEM

The PACRIM 2 TOPSAR DEM data comprised four bands including the C-VV polarised band, the correlation information (DEM accuracy estimate), the incidence angle (degrees) of the radar signal and the calculated topographic DEM. The conversion from the supplied TOPSAR data format into these four parameters was done using ENVITM . Missing radar returns from radar shadows produced by the topography were replaced using standard surface fitting techniques available with ENVITM although the interpolation was still apparent in some areas of these TOPSAR DEM imagery. The resulting four parameters associated with the C band derived DEM comprised identical number of pixel and line samples as the TOPSAR polarimetric file for each flight line and WGS 84 geographic coordinates within the supplied header information (Appendix 1). Consequently the same registration procedure listed in Section 2.1 was followed to generate equivalent GDA94 MGA Zone 55S imagery for the three flight lines.

# 2.3 Issues affecting radar interpretation and quality control

## 2.3.1	 Incident angle

The sensitivity of a radar systems response to the geometry of its operation and the acquisition of returning backscatter is illustrated by Figures 6 and 7. In particular Figure 6 shows the issue of layover distorting the dimensions of the topographic slopes for large radar depression angles, while radar shadowing can be present for shallow depression angles. Typically the AIRSAR/TOPSAR system describe their acquisition specifications in terms of look angle, which is related to the depression (e.g. = 90 – look angle degrees) and incident angles as shown in Figure 7a). In a flat ground surface situation, the look angle and incident angles are equal (Figure 7a) however the local incident angle can be significantly affected by local topography (Figure 7b). When the local incidence angle is smaller than the local slope angle layover effects and distortion will be apparent (Raney, 1998).

The high spatial sampling resolution of 5 metres acquired by the AIRSAR/TOPSAR imagery makes the local incident angle particularly important in the steeply dipping relief of western Tasmania. The range of look angles measured by the AIRSAR/TOPSAR PACRIM2 surveys varied between approximately 25 and 64 degrees. In general the larger look angles acquired by AIRSAR/TOPSAR at the furthest extent of the swath or range (e.g. 50-64 degrees) will generally accentuate geomorphic and structural features related to topographic relief and outcrop by their apparent shadowing effect. At shallower look angles (e.g. 25-35 degrees)  a greater chance of spatial distortion and layover effects is also possible, depending on the local relief.

Figure 6 Schematic diagram illustrating the interaction between topography and the radar depression angle on the resulting radar image effects of layover and shadowing (Elachi & Van Zyl, 2006).

a)						b)

Figure 7 a) Relationship between look angle, depression angle and incident angle assuming a relatively flat ground surface (Ford, 1998); b) Schematic diagram illustrating local incident angle θloc and the average incident angle θi resulting from local slope angle αloc (Raney, 1998).

## 2.3.2	 Surface roughness

In general radar backscatter increases with increasing surface roughness depending on the radar wavelength (Raney, 1998). Geological applications of AIRSAR/TOPSAR utilise these effects of the differing responses to surface roughness by observing the response to the three different band wavelengths. In particular, the surface roughness properties of the rock outcrops and regolith, will generate variable amounts of diffuse backscatter according to the wavelength of the radar pulse and its incident (local) angle. Surface roughness is typically measured in terms of root mean square (rms) measures varying from millimetres , centimetres to metres for very rocky terrain. Quantitatively, a surface is considered definitely “rough” (e.g. generating a high backscatter response) if :

h  > λ / (4.4 Cos θ)						(1)

where h : rms height, λ : radar wavelength, and θ : incident angle (Ford, 1998).

Alternatively a surface is considered definitely “smooth” (e.g. generating a low backscatter response) if :

h  < λ / (25 Cos θ)						(2)

(Ford, 1998).

Assuming the AIRSAR/TOPSAR system parameters for these data sets, the following rms height definitions for “rough” and “smooth” surface roughness were calculated using Eq (1) and Eq. (2) for the minimum and maximum look angles across the radar acquisition swath. No allowance is made for topographic relief and the incident angle, θi, is assumed to be equivalent to θloc and the survey look angle (Figures 7a&b).

Table 1 highlights the potential variation of the AIRSAR/TOPSAR response to look angle across the swath of the imagery (Figure 2) without taking the topographic effects into account. P, and to a lesser extent L radar band, are more likely to observe a “smooth” radar surface at the far swath extent or range of the imagery while C band is most likely to register rough radar surfaces at the near swath range (Table 1). The P band radar imagery is also most likely to discriminate the roughest surface nature at the nearest swath extent of the imagery. The question of whether the surface is a thick vegetation canopy or rocky ground surface is also affected on the ability of the longer wavelength radar to penetrate vegetation and also its signature polarisation. As discussed in Section 1.1 and illustrated in Figure 3, volume scattering measured by P band (and to a lesser extent L band) with a HV polarisation provides the optimal imagery to observe geologically related surface roughness beneath vegetation canopy. HV is also optimal for minimising the effects of variable local incident and look angles (Tapley, 2002).

| AIRSAR/TOPSAR Band | C | L | P |
| --- | --- | --- | --- |
| Wavelength (λ, cm) | 5.7 | 25 | 68 |
| Response at far swath range : |  |  |  |
| "Rough" rms height (cm) @ θi = 68 degrees | >3.5 | >15.2 | >41.3 |
| "Smooth" rms height (cm) @ θi = 68 degrees | <0.6 | <2.7 | <7.3 |
| Response at near swath range : |  |  |  |
| "Rough" rms height (cm) @ θi = 25 degrees | >1.4 | >6.3 | >17.1 |
| "Smooth" rms height (cm) @ θi = 25 degrees | <0.3 | <1.1 | <3.0 |

Table 1 Definitions of smooth and rough radar surfaces for AIRSAR/TOPSAR radar band wavelengths and maximum-minimum range of look angles where Look angle = Incident angle, assuming a flat ground surface.

Studies by Huadong et al., (1996) using shuttleborne C and L band multi-polarisation radar found that L band HV polarised imagery was the best combination for discriminating two different basalt units, alluvium and bedrock. Kierein-Young et al. (1992) showed that quantifiable estimates of surface roughness rms is possible in an arid environment such as Death Valley, Nevada. Height rms values were calculated from modelling the fractal dimensions of the C, L and P band power spectral density however the model assumed generally smooth playa to partly smooth alluvial surfaces (Kierein-Young et al., 1992). Subsequent communication with one of the authors, indicated that in thickly forested environments there are wavelength-specific problems related to partial canopy penetration, leaf moisture dielectric effects and canopy/understory vegetation cover that combine to make this a difficult problem (Kruse, pers. comm.). The observation of surface roughness is also identified by Raney (1998) to be more subtle and less quantifiable with increasing penetration through vegetation cover. Ford (1998) also notes that estimation of surface roughness using multi wavelength radar can be possible within sparsely vegetated terrains.

## 2.2.3	Dielectric properties and Moisture

Another factor that can effects radar backscatter is the dielectric property and related moisture content of the surface materials. Such dielectic properties influence the attenuation and reflectance of the electric field component of the radar pulse. Most rocks exhibit a narrow range in dielectric properties however they will vary significantly as a function of water moisture, its porosity and water holding capacity (Ford, 1998). Water can strongly limit the ground penetration of even the longer wavelength L and P band radar when interacting with moist soil and regolith (Ford, 1998) (Figure 8). As a consequence the radar backscatter response can vary between geological units when they contain different weathered regolith profiles affecting their porosity and moisture content.

Figure 8  Penetration depths for soil with different moisture contents at different wavelengths e.g. C band (λ ~ 3cm) and L band (λ ~ 23cm), after Dobson (1993).

## 2.3.4	 Combined QC issues

The effect of the variable incident angles across each individual flight swaths upon the returned backscatter limits the ability to simply mosaic radar imagery, particularly if flights are in opposite look angle directions (e.g. TS1339 acquired with NE range versus the SW range acquired TS1340 and TS1297). Consequently the presence of radar shadowing and layover will change according to the topographic relief (Section 2.3.1). Such variable look and hence incident angles will also significantly affect the response of surface roughness  across the swath for each AIRSAR wavelength (Section 2.3.2). Changes in vegetation and/or soil moisture due to precipitation between flights could also affect radar backscatter however the acquisition of all three flights on 17/8/2000 makes this a more unlikely influence of image boundary differences. Due to the complexity of these various effects, no attempt has been made to level the radar products.

# 3 Results

## 3.1 TOPSAR DEM

## 3.1.1 Image products

The TOPSAR DEM product processed from the three Tasmanian flight lines appeared to generate good quality data at 5 metre spatial sampling that mosaicked well as a combined DEM data set encompassing 1440 km2 and a range of elevation between 3 to 1370 metres. Examples of the mosaicked DEM product as an image, as a shaded relief (illuminated at 45 degree NE and 60 elevation) and a topographic modelled slope, are shown in Figures 9a, 9b and 9c respectively.

Associated with the supplied TOPSAR DEM product are imagery representing estimates of the local incidence angle (0-180 degrees) to assist in the interpretation of the radar acquisition. Each acquisition will generate a specific incidence  angle  product  relevant for the flight line track and look direction. Figure 10 shows the example of the incidence angle estimates for flight line, TS1340. The lower incident angles shown along the north eastern areas of the imagery generally correspond to the lower radar look angles operating by the AIRSAR/TOPSAR as it flew along a flight path just beyond the NE of the imagery (Figure 10). Although the range of look angle available by the survey is restricted  by the system specification, the effective incidence angle will be dependent according to the local incident angle  as shown in Figure 7b.   As discussed previously, such areas dominated by lower incident angles will be more susceptible to layover distortion while radar shadow effects will increase at the higher incident angles (Figure 10).

A statistical comparison between the overlapping DEM results for the TOPSAR acquisitions (Figure 11) was undertaken to evaluate the consistency of the derived elevations and registration procedure (Appendix 2). Although there appeared spurious variations for the minimum and maximum elevations values, the correlation coefficients were greater than 0.99, and the median / mean values showed a less than 5 metres difference (Appendix 2).

a)							b)							c)

Figure 9 a) TOPSAR DEM mosaic of TS1297, TS1339 and TS1340 acquisitions displayed with a linear 100 to 900 metre elevation stretch; b) Shaded mosaic illuminated with 45o NE azimuth and 60 o  elevation; and c) Slope mosaic product displayed with a linear 0 to 30 degrees slope stretch. All imagery is registered for GDA94 MGA Zone 55

Figure 10 Supplied calculated local incident angle estimates for flight line TS1340.

Figure 11 Depiction of the three TOPSAR acquisitions and their overlapping areas. The spatial extent is the same as shown in Figure 9 a-c).

## 3.1.2 Comparison with SRTM 25m, Photogrammetric derived and LiDar DEMs

Statistical comparisons between the mosaicked TOPSAR DEM product and the MRT supplied SRTM 25 metre, photogrammetric derived and LiDar DEMs was undertaken to assess its quality. Issues of variable spatial sampling resolution and registration between these four datasets is potentially relevant however it was still seen as a useful exercise for validation. ERDAS ERMapperTM software was used to calculate the statistics and subset according to their spatial extents. Table 2 shows the comparison between the TOPSAR SRTM 25 metre, photogrammetric derived DEMs, while Table 3 shows the comparison for the significantly smaller coincident area encompassing all four DEMs including SRTM 25 metre, photogrammetric and LiDar derived DEMs. The relevant LiDar data sets for this comparison were the Meredith, South1 and North1 acquisitions. The statistics for the two coincident areas showed that although the TOPSAR DEM exhibited significantly lower minimum values than the other DEMs, the overall comparison was favourable with a correlation of 0.996 and higher (Table 2 and 3).

|  | TOPSAR | SRTM 25m | Photogrammetry |
| --- | --- | --- | --- |
| Area (Hectares) | 144029 | 144029 | 144029 |
| Spatial sampling (m) | 5 | 25 | 25 |
| Minimum | 3.1 | 54.6 | 50.0 |
| Maximum | 1369.8 | 1246.8 | 1266.6 |
| Mean | 365.1 | 364.2 | 361.4 |
| Median | 334.1 | 329.4 | 325.6 |
| Std.Dev. | 163.4 | 164.0 | 165.9 |
| CorrelationMatrix | TOPSAR | SRTM 25m | Photogrammetry |
| TOPSAR | 1 | 0.997 | 0.996 |
| SRTM25m | 0.997 | 1 | 0.999 |
| Photogrammetry | 0.996 | 0.999 | 1 |

Table 2 Statistical comparison between coincident TOPSAR, SRTM 25m and Photogrammetry derived DEMs.

|  | TOPSAR | SRTM 25m | Photogrammetry | LiDAR |
| --- | --- | --- | --- | --- |
| Area (Hectares) | 17653 | 17653 | 17653 | 17653 |
| Spatial sampling (m) | 5 | 25 | 25 | 0.5 - 2.0 |
| Minimum | 3.5 | 102.6 | 97.0 | 95.0 |
| Maximum | 917.0 | 915.8 | 920.0 | 1160.6 |
| Mean | 434.5 | 437.1 | 434.7 | 434.0 |
| Median | 481.7 | 483.8 | 479.6 | 478.0 |
| Std.Dev. | 166.3 | 162.9 | 164.8 | 164.3 |
| CorrelationMatrix | TOPSAR | SRTM 25m | Photogrammetry | LiDAR |
| TOPSAR | 1 | 0.998 | 0.998 | 0.996 |
| SRTM25m | 0.998 | 1 | 0.999 | 0.998 |
| Photogrammetry | 0.998 | 0.999 | 1 | 0.998 |
| LiDAR | 0.996 | 0.998 | 0.998 | 1 |

Table 3 Statistical comparison between coincident TOPSAR, SRTM 25m, Photogrammetry  and LiDar derived DEMs.

A further analysis was undertaken to assess the accuracy of the elevations and spatial Easting and Northing coordinates of the TOPSAR, relative to the other supplied DEMs and, in particular, to the 1:25,000 topographic mapping. The effect of the geology on the derived DEM elevation values was also briefly examined.  In particular, two statistical training subsets were chosen from the areas with coincident DEM coverage, one encompassing the Meredith intrusives (Dg, Dga), and another with no Meredith intrusives, dominanted by the Mt Read volcanics (Figure 12). The statistical results for each of the two training areas and the entire coincident DEM area are listed in Appendix 3. There again appeared differences between the minimum and maximum DEM values from spurious outliers however the mean and median values were fairly similar within both the Meredith and non Meredith granitic areas. The Meredith Granite area showed a smaller variation in median elevations than the volcanic terrain (e.g. 3.82 m cf 7.06 m) (Appendix 3). The correlation coefficients were generally high for both areas although the volcanic terrain showed higher values (> 0.998) compared to the Meredith granite (> 0.95). However a spurious LiDar maximum of 1160 metres possibly generated this difference.

A detailed comparison between the DEM data sets was also undertaken using five spot topographic peaks within both training areas (Table 4). The supplied 1:25,000 topographic mapping with surveyed elevations were used as a reference to calculate the difference (“Δ(1:25,000)”) relative to the four DEM data sets (Table 4). No significant differences were observed and generally within 50 metres coordinate accuracy and better than 15 metres elevation accuracy. There was potentially subjectivity in choosing the position of the  topographic peaks from the various DEMs which may explain the discrepancy of the Mt Sale Lidar result (Table 4). These results appeared consistent with estimates of spatial x-y registration errors suggested by the Root Mean Square (rms) values obtained for the first order polynomial transformation for each individual TOPSAR flight line (Appendix 1). In particular, rms values ranged from between 4.5 and 9.4 pixels (e.g.  24 to  47 metres) (Appendix 1).

Figure 12 Areas of coincident TOPSAR (blue:low; red:high), SRTM 25m, Photogrammetric 25m and LiDar DEMs overlaying 1:250,000 geology. The Meredith Granite (DG, Dga – red boundary, blue subset) and non-Meredith granite (black subset) highlight the two training areas for statistical comparison. The mosaicked TOPSAR DEM survey area is shown in grey.

| Coordinates (GDA94) | Mt Black | Δ(1:25,000) | Mt Sale | Δ(1:25,000) | Burns Peak | Δ(1:25,000) |  | Δ(1:25,000) |  | Δ(1:25,000) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1:250,000 Geology: | єdv |  | єdv |  | єdsvl |  | Dg/Dga |  | Dg/Dga |  |
| (Easting) 1:25,000 | 380647 |  | 382944 |  | 378385 |  | 363972 |  | 362842 |  |
| (Northing) 1:25,000 | 5375580 |  | 5377974 |  | 5384652 |  | 5394495 |  | 5398980 |  |
| (Elev.) 1:25,000 | 929 |  | 521 |  | 660 |  | 584 |  | 623 |  |
| (Easting) TOPSAR | 380642 | -5 | 382932 | -12 | 378373 | -12 | 363963 | -9 | 362839 | -3 |
| (Northing) TOPSAR | 5375598 | 18 | 5377978 | 4 | 5384656 | 4 | 5394546 | 51 | 5399011 | 31 |
| (Elev.) TOPSAR | 916 | -13 | 532 | 11 | 652 | -8 | 588 | 4 | 621 | -2 |
| (Easting) SRTM25m | 380667 | 20 | 382895 | -49 | 378395 | 10 | 363995 | 23 | 362867 | 25 |
| (Northing) SRTM25m | 5375604 | 24 | 5377940 | -34 | 5384678 | 26 | 5394451 | -44 | 5398991 | 11 |
| (Elev.) SRTM25m | 916 | -13 | 525 | 4 | 653 | -7 | 572 | -12 | 612 | -11 |
| (Easting) Photogram. | 380626 | -21 | 382923 | -21 | 378363 | -22 | 363977 | 5 | 362839 | -3 |
| (Northing) Photogram. | 5375611 | 31 | 5377980 | 6 | 5384652 | 0 | 5394498 | 3 | 5398991 | 11 |
| (Elev.) Photogram. | 920 | -9 | 520 | -1 | 650 | -10 | 580 | -4 | 620 | -3 |
| (Easting) LiDAR | 380644 | -3 | 382848 | -96 | 378401 | 16 | 363991 | 19 | 362857 | 15 |
| (Northing) LiDAR | 5375591 | 11 | 5377960 | -14 | 5384637 | -15 | 5394510 | 15 | 5398970 | -10 |
| (Elev.) LiDAR | 927 | -2 | 528 | 7 | 662 | 2 | 582 | -2 | 621 | -2 |

єdv : Felsic to intermediate calc-alkaline volcanic rocks, (Central Volcanic Complex and correlates / Mt Read Volcanics)

єdsvl : Felsic lava within Western Volcano-Sedimentary Sequence and correlates

Dg/Dga : Meredith granite and associated intrusives

Table 4 Spot comparisons between the TOPSAR, SRTM25m, Photogrammetric 25m and Lidar DEMs with five 1:25,000 topographic located hilltops.

## 3.2 TOPSAR multi-wavelength and multi-polarimetric products

The image products from the three AIRSAR/TOPSAR acquisitions were imported into an ERDAS ERMapperTM algorithm style GIS database incorporating ancillary MRT supplied vector and raster files of other DEMs, SPOT imagery, orthophotography, geology, vegetation, and airborne geophysics. Two areas within TS1340 were studied in particular in order to observe the effects of incident angle, wavelength and polarisation as well as compare with published geological and vegetation information (Figure 13). In particular it was attempted to examine the issues of the potentially competing backscattering effects of vegetation versus geological. Figure 13 also shows the enhanced backscatter of the C band VV polarisation (Cvv) near swath range along the north eastern edge of the image associated with the lower incident angle.

Figure 13 AIRSAR/TOPSAR Cvv image of flight line TS1340 showing the locations of the vegetation (white box) and geological (magenta box) study areas.

## 3.2.1  TS1340 Northern study area

The northern portion of flight line TS1340 proved an useful area for examining the response of the multi -wavelength and -polarisation radar to discriminate vegetation where limited geological outcrop and diversity of units appeared apparent by the published 1:250000 geology (Figure 14a). A range of temperate rainforest and vegetation occupies this area from the steep hills and gullies to swampy floodplains (Figure 14b). Figure 14c highlights this topography and limited geological units mapped within this lowland dominated terrain. The predominant mapped unit is the Cowrie Siltstone intruded by with mostly narrow north-easterly trending Tayatea Dyke Swarm dolerite bodies. There is a significant variation in vegetation from swampy moorland grasses to wet rainforest to open woodland (Figure 14d). Figure 14d also indicates the short wavelength and vertically polarised Cvv band exhibits an exaggerated bias in backscatter related to lower incident angles (e.g. compare with Figure 10).

a)							b)

c)							d)

Figure 14 a) Published 1:250000 geology of the vegetation study area; b) 1:100000 topographic map; c)  shaded SRTM 25m DEM with 1:250000 geology overlay; d) Pseudo coloured Cvv backscatter response (blue:low backscatter return; red high backscatter) with 1:25,000 vegetation overlay. Black hatch: Eucalyptus obliqua wet forest (WOU),  white hatch : Buttongrass moorland (MBU); black stipple : Nothofagus-Phyllocladus short rainforest (rms); black cross hatch : Eucalyptus nitida dry forest and woodland (DNI).

The Lhv band and polarisation (Figure 15) and Phv, showed a better comparison between the 1:25000 vegetation classification than the Cvv band (Figure 14d). There appeared less lower incident angle bias with this longer wavelength cross polarised bands, producing similar backscatter responses for the same vegetation class throughout the imagery. For example the Buttongrass moorland is indicated by a consistently low backscatter return (Figure 15).

Figure 15 Pseudo coloured Lhv backscatter response (blue:low backscatter return; red high backscatter) with 1:25000 vegetation overlay. Black hatch: Eucalyptus obliqua wet forest (WOU),  white hatch : Buttongrass moorland (MBU); black stipple : Nothofagus-Phyllocladus short rainforest (rms); black cross hatch : Eucalyptus nitida dry forest and woodland (DNI).

A closer comparison of the higher resolution TOPSAR  DEM  with SRTM 25m DEM and the published geology reveals that subtle  extensions to the  north easterly trending Tayatea Dyke Swarm dolerite bodies (Figure 16). The improvement in spatial resolution from the 25 metre SRTM (Figure 14c) to the 5 metre airborne TOPSAR (Figure 16) appears to  significantly enhance the potential geological interpretation.

Figure 16 Artificially shaded TOPSAR DEM (45 deg. NE azimuth, 60 deg. elevation)  with 1:250000 published geology. Suggested extensions or presence of dolerite dykes are highlighted within dashed red ellipses.

## 3.2.2  TS1340 Southern study area

The 1:25,0000 topographic mapping of the southern TS1340 area shows a more deeply incised relief with several watercourses including the Donaldson River (Figure 17a). The published 1:25,0000 geology indicates a mixture of Cowrie Siltstone, metasedimentary units, and dolerite dyke intrusives overlain by Tertiary basalts (yellow) (Figure 17b). The radiometrics imagery appears only useful for highlighting the Cowrie Siltstone and Balfour Subgroup (Figure 17c) while the aeromagnetics shows the importance of the north and northeast trending structures and boundaries (Figure 17d).

a)							b)

c)							d)

Figure 17 a) 1:25,0000 topographic map of TS1340 southern study area as shown in Figure 13; b) 1:2500000 published geology with Tertiary basalt highlighted in yellow; c) Radiometric Ternary image (RGB : KThU) with geology overlay. Tertiary basalt highlighted within red hatched area; d) High frequency Tilt filtered aeromagnetics. Tertiary basalt highlighted within red hatched area.

The TOPSAR DEM product (Figure 18b) again shows a significant improvement over the SRTM 25 DEM (Figure 18a). Subtle north trending elevation features appear to be associated with dolerite dykes and their possible extension (Figure 18b).

Figure 18 a)  Shaded SRTM 25m DEM (45 deg. NE azimuth, 60 deg. elevation) with 1:25,0000 geology overlay. Tertiary basalt highlighted within red hatched area. b) Shaded TOPSAR DEM (45 deg. NE azimuth, 60 deg. elevation) with 1:25,0000 published geology overlay. Tertiary basalt highlighted within red hatched area.

An examination of the various multi-wavelength and -polarimetric bands with the published geology indicated that optimal results were obtained with Phv imagery (Figure 19). A north westerly fault line transecting the Balfour Subgroup sedimentary units (“A”) was clear within the DEMs and the Phv imagery (Figure 19). There also appeared to be a Phv anomalous area encompassing and larger than the presently mapped dolerite Tayatea Dyke nearby (“B”) although this response wasn’t replicated elsewhere (Figure 19). The overlying Tertiary basalt shows a consistently moderate to high Phv backscatter within areas shown by hatching (“C”) (Figure 19). The Arthur Metamorphic Complex unit appears to have the highest surface roughness related backscatter (“D”) while the Cowrie Siltstone shows a generally low backscatter  (“E”) (Figure 19). The same area also has a diversity of vegetation types (Figure 20). Anomalous areas, “B” and “D” shown in Figure 19, also appear to have classified vegetation boundaries associated with them (Figure 20). It is possible that geobotanical associations coexist with these geological units. Further ground truthing which assist an understanding of such geological-vegetation spatial relationships.

RGB colour composite imagery combining the various bands and polarisations did not generally produce imagery that clearly discriminated either vegetation or geology. A RGB image consisting of Phh, Pvv and Phv showed some subtle variations in hue across the various units (Figure 21). The greatest benefit appeared to be the highlighting of topographic features, possibly related to geological structure or lineaments (Figure 21).

Figure 19 Pseudo coloured Phv backscatter response (blue:low backscatter return; red high backscatter) with 1:250000 geology overlay. Tertiary basalt highlighted within hatched area.

Figure 20 Pseudo coloured Phv backscatter response (blue:low backscatter return; red high backscatter) with 1:25,000 vegetation overlay.

Figure 21 RGB composite image of Phh, Pvv, Phv backscatter response with 1:25,0000 geology overlay.

## 3.2.3  Mosaicked polarimetric imagery

Mosaicked results combining the three flight lines of multi-wavelength and -polarimetric data were affected by the issues described in Section  2.3.4. RGB colour composite products for  the combined Cvv, Ltp and Ptp  products showed that a uniform linear stretch for the three acquisitions produced such boundary issues (Figure 22a). However the application of independently calculated 99 percent  linear stretching for each acquisition generally improved the mosaicked product (Figure 22b). This effect was most notable for bands such as the Cvv (Figure 23a).  An improvement in mosaicked results were generated using Phv and gave similar results whether an uniform or independent flight line linear stretching was applied (Figure 23b). This is consistent with the description earlier that the cross-polarized  HV scattering  bands are less dependent on incidence angle (and therefore, terrain),  than the co-polarized (HH and VV) backscatter (Tapley, 2002).

a)										b)

Figure 22 a) RGB composite of Cvv, Ltp and Ptp with uniform linear stretch applied to each band of combined TOPSAR acquisitions; b)  RGB composite of Cvv, Ltp and Ptp with 99.0% linear stretch applied independently to each band of TS1297, TS1339 and TS1340. All imagery is registered to GDA94 MGA Zone 55.

a)									b)

Figure 23 a) Greyscale imagery of Cvv with 99.0% linear stretch applied independently to each band of TS1297, TS1339 and TS1340; b)  Greyscale imagery of Phv with uniform linear stretch applied to combined TOPSAR Phv. All imagery is registered to GDA94 MGA Zone 55.

# 4 Conclusions

This study demonstrated the successful processing and application of TOPSAR DEM and associated multi –wavelength and –polarimetric data for geoscience information in western Tasmania. In particular, a preliminary examination of the higher resolution DEM product showed its potential as a  superior tool for topographic mapping and geological interpretation than available SRTM 25 imagery. A comparison of the TOPSAR DEM product with the 1:25,000 Tasmanian topographic mapping and other data sets revealed a high level of  spatial and elevation accuracy. The DEM accuracy of the airborne TOPSAR data also appeared to provide consistent results, independent of the underlying geology. Although the terrain is generally forested with a variety of vegetation types, P band with a HV polarisation shows potential to map a range of surface roughness that may be related to geological units and their associated regolith. However it is recommended that the detailed 1:25,000 vegetation classification be used with any interpretation with this radar data. The complexity of radar scattering within rainforest canopies, vegetated understories and a damp regolith is likely to generate, at least partially, non-unique solutions to the question of geological versus vegetation controls. Also, an understanding of potential geobotanical associations and geologically defined variations in surface roughness would be further assisted with field work and ground truthing. Further processing of the TOPSAR data for estimates of effective layover distortion and radar shadowing using DEM information would also assist the interpretation.

# 5 Acknowledgements

This study has been kindly supported and funded by Mineral Resources Tasmania (MRT), with a considerable help from MRT’s David Green. Professor Simon Jones of the School of Mathematical & Geospatial Sciences within RMIT University has also been supportive of this project. Technical assistance by Phil Wilkes, PhD candidate, has been forthcoming particularly in regard to handling LiDar data sets. Software support from Laurie Buxton of RMIT University was also helpful during this study.

# 6 References

Elachi, C. & Jakob J. van Zyl, J., (2006), “Introduction To The Physics and Techniques of Remote Sensing”, 2nd Edition,John Wiley & Sons, New Jersey, ISBN: 978-0-471-47569-9.

ERDAS, (1999), “ERDAS Field Guide”, Fifth Edition, ERDAS Inc., Atlanta, Georgia, http://www.gis.usu.edu/manuals/labbook/erdas/manuals/FieldGuide.pdf).

Evans, L., Farr, T.G., Van Zyl, J.J., and Zebker, H.A., (1988) , “ Radar Polarimetry: Analysis Tools and Applications, IEEE Transactions on Geoscience and Remote Sensing,  Vol. 26, No.6, pp. 774-789.

Ford, J.P., 1998, Radar Geology, in Principles and Application of Imaging Radar (F.M. Henderson and A.J. Lewis, eds.), Manual of Remote Sensing, Third Edition, Volume 2, John Wiley and Sons, Inc.

Hewson, R.D. and Taylor, G.R., (2000). “An investigation of the geological and geomorphological features of Fowlers Gap using thermal infrared, radar and airborne geophysical remote sensing techniques.” Rangeland Journal, Vol. 22, pp. 105-123.

Huadong, G., Jingjuan, L., Changlin, W., Chao, W., Farr, T., Evans, D.L., (1996), “Use of Multifrequency, Multipolarisation Shuttle, Imaging Radar for Volcano mapping in the Kunlun Mountains of western China”, Remote Sensing of Environment, Vol. 59, pp.364-374.

Kierein-Young, K.S., Kruse, F.A., and Lefkoff, A.B., (1992), “Quantitative Analysis of Surface Characteristics and Morphology in Death Valley, California, using AIRSAR data, In Proc. Geoscience and Remote Sensing Symposium, 2002, Houston, IEEE, pp. 392-394, DOI: 10.1109/IGARSS.1992.576720

Lee,  Jong-Sen, “Digital Image Enhancement and Noise Filtering by Use of Local Statistics”, (1980), Pattern Analysis and Machine Intelligence, IEEE Transactions on Pattern Analysis and Machine Intelligence, Volume: PAMI-2 , Issue: 2, pp 165-168.  DOI: 10.1109/TPAMI.1980.4766994

Lou, Yunling (2002), “Review of the NASA/JPL airborne synthetic aperture radar system,”

In Proc. Geoscience and Remote Sensing Symposium, 2002. IEEE, Vol. 3, pp. 1702 – 1704, DOI: 10.1109/IGARSS.2002.1026226

Lopes, A. ; Touzi, R. ; Nezry, E., (1990), “Adaptive speckle filters and scene heterogeneity”, IEEE Transactions on Geoscience and Remote Sensing,  Vol. 28 , No.6, pp. 992 – 1000.  DOI: 10.1109/36.62623.

Madsen, S.N., Zebker, H.A., and Martin, J., (1993), “Topographic Mapping using Radar Interferometry: Processing Techniques,“ IEEE Transactions on Geoscience and Remote Sensing,  Vol. 31, No. 1, pp. 246-256.

Raney, R.K., 1998, Radar fundamentals: technical perspective, in Principles and Application of Imaging Radar (F.M. Henderson and A.J. Lewis, eds.), Manual of Remote Sensing, Third Edition, Volume 2, John Wiley and Sons, Inc.

Tapley, I.J. (2002), “Radar Imaging” within  “Geophysical and remote sensing methods for  regolith  exploration”, Ed. Eva Pap, CRC LEME Open file report 144, http://crcleme.org.au/Pubs/OPEN%20FILE%20REPORTS/OFR%20144/05Radar.pdf

Zebker, H.A., Madsen, S.N., Martin, J., Wheeler, K.B., Miller, T., Lou, Yunling, Alberti, G., Vetrella, S.  and Cucci, A., (1992), “The TOPSAR Interferometric Radar Topographic Mapping Instrument,” IEEE Transactions on Geoscience and Remote Sensing,  Vol. 30 , No. 5, pp. 933 – 940.

# 7 Appendix 1

# Registration ground control points of TOPSAR imagery

Note : all imagery was registered to GDA94 MGA Zone 55.

Figure A1 Shaded TOPSAR DEM with location of ground control points used for registration of each of the three TOPSAR flight lines, TS 1297, 1339 and 1340.

TS1297 :

; ENVI Image to Map GCP File

; projection info = {Geographic Lat/Lon, WGS-84, units=Degrees}

; warp file: C:\Tassie_MRT_AIRSAR-TOPSAR\ts1297\ts1297_c_dem_bad-value-remove

; Map (x,y), Image (x,y)

;

145.73019400      -41.85010100          0.000000          0.000000

145.61782800      -41.91090400       2284.000000          1.000000

145.21191400      -41.49717700       2284.000000      11431.000000

145.32418800      -41.43636300          0.000000      11431.000000

; ENVI Image to Map GCP File

; projection info = {3, 6378137.0, 6356752.3, 0.000000, 147.000000, 500000.0, 10000000.0, 0.999600, Geocentric Datum of Australia 1994, Map Grid of Australia (MGA 94) Zone 55, units=Meters}

; warp file: G:\Tassie_MRT_AIRSAR-TOPSAR\ts1297\ts1297_c_dem_bad-value-remove_MGA55_RST_shade45Az60El

; Map (x,y), Image (x,y)

;

354469.7916      5402228.9750        621.000000       1920.000000

364394.7916      5394028.9750       2688.000000       3540.000000

377069.7916      5382953.9750       5305.000000       5721.000000

376819.7916      5389503.9750       5263.000000       4404.000000

372644.7916      5387078.9750       4390.000000       4886.000000

370444.7916      5381303.9750       3940.000000       6062.000000

359594.7916      5404203.9750       1698.000000       1516.000000

379619.7916      5376903.9750       5838.000000       6915.000000

382194.7916      5378953.9750       6365.000000       6500.000000

384844.7916      5373278.9750       6923.000000       7625.000000

363219.7916      5400028.9750       2440.000000       2332.000000

365819.7916      5389528.9750       2980.000000       4435.000000

369094.7916      5388453.9750       3655.000000       4638.000000

375244.7916      5385153.9750       4931.000000       5288.000000

380019.7916      5380378.9750       5911.000000       6232.000000

374794.7916      5379803.9750       4839.000000       6351.000000

377994.7916      5378353.9750       5502.000000       6630.000000

378644.7916      5369953.9750       5648.000000       8301.000000

381344.7916      5367628.9750       6189.000000       8776.000000

390019.7916      5370103.9750       7990.000000       8263.000000

392469.7916      5366803.9750       8480.000000       8907.000000

389694.7916      5363978.9750       7915.000000       9478.000000

381244.7916      5365153.9750       6175.000000       9257.000000

386894.7916      5364753.9750       7329.000000       9310.000000

356969.7916      5400528.9750       1139.000000       2255.000000

362344.7916      5394103.9750       2251.000000       3521.000000

367994.7916      5401178.9750       3430.000000       2089.000000

377977.3770      5378879.3262       5494.000000       6524.000000

376917.0650      5372954.3391       5271.000000       7703.000000

355903.0313      5404300.5233        903.000000       1494.000000

372451.9260      5388001.5225       4345.000000       4708.000000

369902.5303      5383505.9756       3820.000000       5614.000000

Note : Root Mean Square (rms) error of registeration : 9.4 pixels

TS1339 :

; ENVI Image to Map GCP File

; projection info = {3, 6378137.0, 6356752.3, 0.000000, 147.000000, 500000.0, 10000000.0, 0.999600, Geocentric Datum of Australia 1994, Map Grid of Australia (MGA 94) Zone 55, units=Meters}

; warp file: C:\Tassie_MRT_AIRSAR-TOPSAR\ts1339\ts1339_c_demi2_convert_DEM_masked

; Map (x,y), Image (x,y)

;

320409.6464      5434507.1693          0.000000          0.000000

329478.6014      5441404.1975       2288.000000          1.000000

366965.1318      5392380.5798       2288.000000      12273.000000

357936.3988      5385527.6382          1.000000      12273.000000

; ENVI Image to Map GCP File

; projection info = {3, 6378137.0, 6356752.3, 0.000000, 147.000000, 500000.0, 10000000.0, 0.999600, Geocentric Datum of Australia 1994, Map Grid of Australia (MGA 94) Zone 55, units=Meters}

; warp file: G:\Tassie_MRT_AIRSAR-TOPSAR\ts1339\ts1339_c_demi2_convert_DEM_masked_MGA55_RST_Shade135Az60El

; Map (x,y), Image (x,y)

;

322869.7916      5433303.9750        511.000000       1748.000000

330869.7916      5432428.9750       2150.000000       1927.000000

328744.7916      5439603.9750       1703.000000        504.000000

331369.7916      5424828.9750       2276.120000       3448.160000

334870.0000      5417978.9750       2994.000000       4808.000000

342344.7916      5420228.9750       4526.000000       4350.000000

344616.4647      5416774.9417       4987.000000       5044.000000

340919.7916      5410978.9750       4255.000000       6207.000000

344394.7916      5405428.9750       4978.000000       7321.000000

349869.7916      5409503.9750       6078.000000       6498.000000

348069.7916      5400478.9750       5731.000000       8319.000000

354694.7916      5405978.9750       7073.000000       7223.000000

351969.7916      5393503.9750       6538.000000       9723.000000

358469.7916      5399653.9750       7863.000000       8486.000000

354469.7916      5395753.9750       7039.000000       9268.000000

357494.7916      5388328.9750       7670.000000      10752.000000

363794.7916      5392478.9750       8939.000000       9925.000000

357694.7916      5394253.9750       7703.000000       9563.000000

346519.7916      5412378.9750       5395.360000       5933.160000

335925.4663      5429417.0885       3188.640000       2522.540000

339866.1821      5424782.7668       4011.000000       3447.000000

326893.2004      5431585.2948       1337.000000       2089.000000

Note : Root Mean Square (rms) error of registeration : 7.3 pixels

TS1340 :

; ENVI Image to Map GCP File

; projection info = {Geographic Lat/Lon, WGS-84, units=Degrees}

; warp file: C:\Tassie_MRT_AIRSAR-TOPSAR\ts1340\ts1340_c-demi2_convert

; Map (x,y), Image (x,y)

;

145.31069900      -41.41519900          1.000000          1.000000

145.20140100      -41.47602800       2286.000000          1.000000

144.98007200      -41.25043100       2286.000000       6234.000000

145.08926400      -41.18959000          1.000000       6234.000000

; ENVI Image to Map GCP File

; projection info = {3, 6378137.0, 6356752.3, 0.000000, 147.000000, 500000.0, 10000000.0, 0.999600, Geocentric Datum of Australia 1994, Map Grid of Australia (MGA 94) Zone 55, units=Meters}

; warp file: C:\Tassie_MRT_AIRSAR-TOPSAR\ts1340\ts1340_c-demi2_convert_DEM-masked_MGA55_Shade_60El135Az

; Map (x,y), Image (x,y)

;

343319.7916      5420078.9750       2505.000000       3678.000000

347344.7916      5424928.9750       3300.000000       2704.000000

351644.7916      5418978.9750       4203.000000       3873.000000

349844.7916      5409553.9750       3875.000000       5757.000000

353519.7916      5410953.9750       4612.000000       5479.000000

355694.7916      5413178.9750       5045.000000       5033.000000

335929.0870      5429415.7599        964.000000       1830.000000

342740.8529      5434363.8219       2329.000000        824.000000

339771.1342      5435805.9494       1724.000000        545.000000

340282.2807      5423896.7677       1871.000000       2919.000000

341771.3644      5429013.9942       2155.000000       1893.000000

350144.7916      5423153.9750       3877.000000       3055.000000

348336.0057      5413514.2958       3547.000000       4971.000000

351723.9351      5416330.8849       4221.000000       4401.000000

356094.7916      5416303.9750       5114.000000       4402.000000

346769.7916      5413753.9750       3234.000000       4936.000000

335413.7162      5431249.0803        846.000000       1456.000000

336999.9600      5426133.9684       1190.170000       2478.550000

346344.7916      5418528.9750       3121.000000       3980.000000

340872.4542      5431167.4678       1962.000000       1466.000000

Note : Root Mean Square (rms) error of registeration : 4.8 pixels

# 7 Appendix 2

# Statistical comparison between DEM results from TOPSAR acquisitions

TS1339 vs TS1340 :

| STATISTICS FOR DATASET: Tas_TOPSAR_DEM-1339_1340_intersected.ers |  |  |
| --- | --- | --- |
| REGION: All |  |  |
|  | TS1339 | TS1340 |
|  | ----- | ----- |
| NullCells | 104081846 | 104081846 |
| Non-NullCells | 5164904 | 5164904 |
| AreaInHectares | 12912.26 | 12912.26 |
| AreaInAcres | 31906.892 | 31906.892 |
| Minimum | 71.364 | 49.9 |
| Maximum | 555.983 | 719.514 |
| Mean | 319.281 | 320.428 |
| Median | 311.781 | 311.468 |
| Std.Dev. | 92.952 | 92.556 |
| Std.Dev.(n-1) | 92.952 | 92.556 |
| Corr.Eigenval. | 1.991 | 0.009 |
| Cov.Eigenval. | 17131.032 | 75.733 |
| CorrelationMatrix | TS1339 | TS1340 |
| ------------------ | ----- | ----- |
| TS1339 | 1 | 0.991 |
| TS1340 | 0.991 | 1 |
| Determinant | 0.018 |  |
| Corr.Eigenvectors | PC1 | PC2 |
| ------------------ | ----- | ----- |
| TS1339 | 0.707 | 0.707 |
| TS1340 | 0.707 | -0.707 |
| Inv.ofCorr.Ev. | PC1 | PC2 |
| ----------------- | ----- | ----- |
| TS1339 | 0.707 | 0.707 |
| TS1340 | 0.707 | -0.707 |
| CovarianceMatrix | TS1339 | TS1340 |
| ----------------- | ----- | ----- |
| TS1339 | 8640.139 | 8527.57 |
| TS1340 | 8527.57 | 8566.626 |
| Determinant | 1297392.466 |  |
| Cov.Eigenvectors | PC1 | PC2 |
| ----------------- | ----- | ----- |
| TS1339 | 0.709 | 0.706 |
| TS1340 | 0.706 | -0.709 |
| Inv.ofCov.Ev. | PC1 | PC2 |
| ---------------- | ----- | ----- |
| TS1339 | 0.709 | 0.706 |
| TS1340 | 0.706 | -0.709 |

TS1297 vs TS1339 :

STATISTICS FOR DATASET: Tas_TOPSAR_DEM-1297-1339_intersected.ers. REGION:All. TS1297; TS1339. -----; -----. NullCells; 250922347; 250922347. Non-NullCells; 3200488; 3200488. AreaInHectares; 8001.43; 8001.43. AreaInAcres; 19771.965; 19771.965. Minimum; 4.619; 128.584. Maximum; 833.887; 834.257. Mean; 505.289; 501.541. Median; 542.347; 542.064. Std.Dev.; 132.846; 135.511. Std.Dev.(n-1); 132.846; 135.511. Corr.Eigenval.; 1.995; 0.005. Cov.Eigenval.; 35916.763; 94.498. CorrelationMatrix; TS1297; TS1339. ------------------; -----; -----. TS1297; 1; 0.995. TS1339; 0.995; 1. Determinant; 0.01. Corr.Eigenvectors; PC1; PC2. ------------------; -----; -----. TS1297; 0.707; 0.707. TS1339; 0.707; -0.707. Inv.ofCorr.Ev.; PC1; PC2. -----------------; -----; -----. TS1297; 0.707; 0.707. TS1339; 0.707; -0.707. CovarianceMatrix; TS1297; TS1339. -----------------; -----; -----. TS1297; 17648.111; 17907.564. TS1339; 17907.564; 18363.15. Determinant; 3394068.61. Cov.Eigenvectors; PC1; PC2. -----------------; -----; -----. TS1297; 0.7; -0.714. TS1339; 0.714; 0.7. Inv.ofCov.Ev.; PC1; PC2. ----------------; -----; -----. TS1297; 0.7; 0.714. TS1339; -0.714; 0.7.

# 7 Appendix 3

## Statistical comparison between DEM results from TOPSAR (TS1297), SRTM25m, Photogrammetric 25m and LiDar (Meredith)

| STATISTICS FOR DATASET: Tas_DEM_analysis4b.ers. REGION: Meredith Granite. TOPSAR; SRTM25m; Photo25m; LiDAR. -----; -----; -----; -----. Null cells; 75923; 75923; 75923; 75923. Non-Null cells; 70578035; 70578035; 70578035; 70578035. Area(Ha); 7057.863; 7057.863; 7057.863; 7057.863. Area(acres); 17440.361; 17440.361; 17440.361; 17440.361. Minimum; 309.724; 322.547; 313.056; 312.168. Maximum; 799.251; 718.522; 730; 1160.635. Mean; 564.214; 562.759; 562.079; 561.317. Median; 567.873; 565.391; 567.131; 564.056. Std.Dev; 48.533; 48.441; 51.048; 51.886. Std.Dev(n-1); 48.533; 48.441; 51.048; 51.886. Corr.Eigenval.; 3.922; 0.053; 0.017; 0.008. Cov.Eigenval.; 9802.053; 137.109; 40.746; 20.148. CorrelationEigenVector; TOPSAR; SRTM25m; Photo25m; LiDAR. ------------------; -----; -----; -----; -----. TOPSAR; 1; 0.984; 0.984; 0.953. SRTM25m; 0.984; 1; 0.992; 0.965. Photo25m; 0.984; 0.992; 1; 0.967. LiDAR; 0.953; 0.965; 0.967; 1. Determinant; 0. Corr.EigenVector; PC1; PC2; PC3; PC4. ------------------; -----; -----; -----; -----. TOPSAR; 0.5; -0.454; 0.737; -0.022. SRTM25m; 0.502; -0.211; -0.491; -0.68. Photo25m; 0.503; -0.173; -0.425; 0.733. LiDAR; 0.495; 0.848; 0.186; -0.032. Inv.; PC1; PC2; PC3; PC4. -----------------; -----; -----; -----; -----. TOPSAR; 0.5; 0.502; 0.503; 0.495. SRTM25m; -0.454; -0.211; -0.173; 0.848. Photo25m; 0.737; -0.491; -0.425; 0.186. LiDAR; -0.022; -0.68; 0.733; -0.032. Covariance; TOPSAR; SRTM25m; Photo25m; LiDAR. -----------------; -----; -----; -----; -----. TOPSAR; 2355.497; 2313.223; 2438.293; 2399.021. SRTM25m; 2313.223; 2346.567; 2452.582; 2424.755. Photo25m; 2438.293; 2452.582; 2605.882; 2560.855. LiDAR; 2399.021; 2424.755; 2560.855; 2692.11. Determinant; 1103315978. Cov.; TOPSAR; SRTM25m; Photo25m; LiDAR. -----------------; -----; -----; -----; -----. TOPSAR; 0.485; -0.446; 0.752; 0. SRTM25m; 0.487; -0.228; -0.449; -0.714. Photo25m; 0.513; -0.206; -0.453; 0.7. LiDAR; 0.514; 0.841; 0.167; -0.022. Inv.; PC1; PC2; PC3; PC4. ----------------; -----; -----; -----; -----. TOPSAR; 0.485; 0.487; 0.513; 0.514. SRTM25m; -0.446; -0.228; -0.206; 0.841. Photo25m; 0.752; -0.449; -0.453; 0.167. LiDAR; 0; -0.714; 0.7; -0.022. |  |  |  |
| --- | --- | --- | --- |
| STATISTICS FOR DATASET: Tas_DEM_analysis4b_no_Meredith.ers. REGION: No_Meredith_Granite. TOPSAR; SRTM25m; Photo25m; LiDAR. -----; -----; -----; -----. Non-NullCells; 85406797; 85406797; 85406797; 85406797. AreaInHectares; 8540.752; 8540.752; 8540.752; 8540.752. AreaInAcres; 21104.66; 21104.66; 21104.66; 21104.66. Minimum; 3.531; 105.086; 97; 94.982. Maximum; 917.031; 915.842; 920; 928.798. Mean; 335.072; 341.235; 337.936; 337.196. Median; 321.115; 326.777; 318.824; 319.721. Std.Dev.; 160.957; 159.243; 160.217; 159.258. Std.Dev.(n-1); 160.957; 159.243; 160.217; 159.258. Corr.Eigenval.; 3.995; 0.003; 0.001; 0.001. Cov.Eigenval.; 102182.685; 69.109; 29.725; 16.677. CorrelationMatrix; TOPSAR; SRTM25m; Photo25m; LiDAR. ------------------; -----; -----; -----; -----. TOPSAR; 1; 0.998; 0.998; 0.998. SRTM25m; 0.998; 1; 0.999; 0.999. Photo25m; 0.998; 0.999; 1; 0.999. LiDAR; 0.998; 0.999; 0.999; 1. Determinant; 0. Corr.Eigenvectors; PC1; PC2; PC3; PC4. ------------------; -----; -----; -----; -----. TOPSAR; 0.5; -0.822; -0.252; -0.103. SRTM25m; 0.5; 0.028; 0.86; 0.102. Photo25m; 0.5; 0.334; -0.384; 0.701. LiDAR; 0.5; 0.46; -0.223; -0.699. Inv.ofCorr.Ev.; PC1; PC2; PC3; PC4. -----------------; -----; -----; -----; -----. TOPSAR; 0.5; 0.5; 0.5; 0.5. SRTM25m; -0.822; 0.028; 0.334; 0.46. Photo25m; -0.252; 0.86; -0.384; -0.223. LiDAR; -0.103; 0.102; 0.701; -0.699. CovarianceMatrix; TOPSAR; SRTM25m; Photo25m; LiDAR. -----------------; -----; -----; -----; -----. TOPSAR; 25907.312; 25587.659; 25736.169; 25574.251. SRTM25m; 25587.659; 25358.315; 25484.135; 25331.676. Photo25m; 25736.169; 25484.135; 25669.439; 25498.565. LiDAR; 25574.251; 25331.676; 25498.565; 25363.13. Determinant; 3500760358. Cov.Eigenvectors; PC1; PC2; PC3; PC4. -----------------; -----; -----; -----; -----. TOPSAR; 0.503; -0.822; -0.248; -0.102. SRTM25m; 0.498; 0.033; 0.86; 0.105. Photo25m; 0.501; 0.337; -0.388; 0.697. LiDAR; 0.498; 0.459; -0.22; -0.702. Inv.ofCov.Ev.; PC1; PC2; PC3; PC4. ----------------; -----; -----; -----; -----. TOPSAR; 0.503; 0.498; 0.501; 0.498. SRTM25m; -0.822; 0.033; 0.337; 0.459. Photo25m; -0.248; 0.86; -0.388; -0.22. LiDAR; -0.102; 0.105; 0.697; -0.702. REGION:All (Inc. Meredith & non-Meredith intrusives) :. TOPSAR; SRTM25m; Photo25m; LiDAR. -----; -----; -----; -----. NullCells; 1206461396; 1206461396; 1206461396; 1206461396. Non-NullCells; 176478100; 176478100; 176478100; 176478100. AreaInHectares; 17647.96; 17647.96; 17647.96; 17647.96. AreaInAcres; 43609.061; 43609.061; 43609.061; 43609.061. Minimum; 3.531; 102.597; 97; 94.982. Maximum; 917.031; 915.842; 920; 1160.635. Mean; 434.5; 437.099; 434.686; 434.036. Median; 481.691; 483.805; 479.566; 477.951. Std.Dev.; 166.346; 162.863; 164.762; 164.268. Std.Dev.(n-1); 166.346; 162.863; 164.762; 164.268. Corr.Eigenval.; 3.993; 0.004; 0.002; 0.001. Cov.Eigenval.; 108145.165; 106.368; 47.345; 27.093. CorrelationMatrix; TOPSAR; SRTM25m; Photo25m; LiDAR. ------------------; -----; -----; -----; -----. TOPSAR; 1; 0.998; 0.998; 0.996. SRTM25m; 0.998; 1; 0.999; 0.998. Photo25m; 0.998; 0.999; 1; 0.998. LiDAR; 0.996; 0.998; 0.998; 1. Determinant; 0. Corr.Eigenvectors; PC1; PC2; PC3; PC4. ------------------; -----; -----; -----; -----. TOPSAR; 0.5; -0.748; 0.435; 0.042. SRTM25m; 0.5; -0.023; -0.55; -0.669. Photo25m; 0.5; 0.117; -0.443; 0.734. LiDAR; 0.5; 0.653; 0.558; -0.108. Inv.ofCorr.Ev.; PC1; PC2; PC3; PC4. -----------------; -----; -----; -----; -----. TOPSAR; 0.5; 0.5; 0.5; 0.5. SRTM25m; -0.748; -0.023; 0.117; 0.653. Photo25m; 0.435; -0.55; -0.443; 0.558. LiDAR; 0.042; -0.669; 0.734; -0.108. CovarianceMatrix; TOPSAR; SRTM25m; Photo25m; LiDAR. -----------------; -----; -----; -----; -----. TOPSAR; 27670.998; 27033.976; 27342.512; 27220.512. SRTM25m; 27033.976; 26524.266; 26805.697; 26696.373. Photo25m; 27342.512; 26805.697; 27146.611; 27016.629. LiDAR; 27220.512; 26696.373; 27016.629; 26984.095. Determinant; 14755001395. Cov.Eigenvectors; PC1; PC2; PC3; PC4. -----------------; -----; -----; -----; -----. TOPSAR; 0.505; -0.75; 0.424; 0.046. SRTM25m; 0.495; -0.014; -0.542; -0.679. Photo25m; 0.501; 0.124; -0.455; 0.726. LiDAR; 0.499; 0.649; 0.565; -0.101. Inv.ofCov.Ev.; PC1; PC2; PC3; PC4. ----------------; -----; -----; -----; -----. TOPSAR; 0.505; 0.495; 0.501; 0.499. SRTM25m; -0.75; -0.014; 0.124; 0.649. Photo25m; 0.424; -0.542; -0.455; 0.565. LiDAR; 0.046; -0.679; 0.726; -0.101. |  |  |