# Appendix 2: Zircon sample preparation, analyses, and data reduction methods

# Sample treatment to extract and mount zircon crystals

The samples were first washed to remove any surface contamination and again to remove grease and/or contamination from the saw. Then, the samples were cut into blocks about the size of a large fist using the saw at CODES/ES lapidary, University of Tasmania in preparation to fit into the crushing machine. In the CODES/ES laboratory, a hydraulic press crushed the samples into chips less than 5 cm across in preparation for milling. Before and after each sample the equipment was blasted with air and wiped down to prevent cross contamination. In the same laboratory, a chrome-steel mill was used to mill the samples to the coarseness of a fine sand. In between each sample the mill was also blasted with air and wiped down to prevent contamination.

Each milled sample was panned to remove the light grains and heavy minerals were transferred to a watch glass. The grains were further panned in the watch glass to separate out zircons from other heavy minerals. Next, each watch glass was put into an oven at approximately 50 degrees for over 30 minutes to dry out. A magnet was then used to remove any magnetic grains which were kept separately, and zircons were transferred to paper envelope for storage. Double-sided sticky tape for each sample was placed onto a slide and zircons were poured onto tape. Epoxy resin was then poured to create the mounts.

Once the epoxy was hardened the surfaces were polished and sent to the Central Science Laboratory (CSL) at the University of Tasmania for carbon coating for imaging. Cathodoluminescence imaging (CL) was conducted at CSL using the Hitachi SU-70 field emission analytical scanning electron microscope of a section of each samples. Results of CL imaging are presented in Appendix 3.

# Obtaining zircon geochemistry

Zircon crystals were analysed using Laser Ablation Inductively Coupled Plasma Mass Spectrometry (LA-ICPMS) equipment in the ES/CODES Analytical Laboratory, University of Tasmania. An ASI Resolution S-155 ablation system with Coherent Complex Pro 110 Ar-F excimer laser working at a 193nm wavelength and a pulse width of 20ns was used for the samples’ analyses. Ablated material was analysed using the Agilent 7900 quadrupole ICPMS. Thompson et al. (2018) describes in detail the method and other condition used for the analyses.

For the detrital zircon the following standards used were before and after every 23 spots:

• 91500 (Wiedenbeck et al., 1995, 2005) was used as the primary calibration standard for the

Pb/U and Pb/Th ratios;

• NIST610 (reference values from http://georem.mpch-mainz.gwdg.de was used to calibrate

the 207Pb/206 Pb ratios and to calibrate the trace element concentrations using Zr as the internal standard element, assuming stoichiometric proportions;

• Plesovice and Temora (Sláma et al., 2008; Black et al., 2003) were used as check standards to

ensure that the ages calculated were accurate;

• GSD-IG and BCR-2G were analysed as check standards for the trace elements and for the Pb

isotope ratios (reference values from http://georem.mpch-mainz.gwdg.de);

Analyses targeted the centre of small zircons and the rims of larger zircons.  Some of the larger zircons had more than one spot. Zircons were ablated using a 20-micron laser beam size, firing frequency at 5 Hz and a laser beam energy density of ~ 2 J/cm2. With each laser ablation analysis, the machine began with a blank gas measurement for 30 seconds to measure baseline elements which was then followed by 20 seconds where the laser was in use. A flow of He carrier gas at a rate of 0.35 litres/minute was mixed with Ar gas carried particles from the ablated zircon out of the chamber and to the plasma torch. The isotopes which were measured included 31P, 49Ti, 56Fe, 89Y, 91Zr, 93Nb, 139La, 140Ce, 141Pr, 146Nd, 147Sm, 153Eu, 157Gd, 159Tb, 163Dy, 165Ho, 166Er, 169Tm, 172Yb, 175Lu, 178Hf, 181Ta, 204Pb, 206Pb, 207Pb, 208Pb, 232Th, 235U and 238U. Each element was measured for approximately 20 seconds, however, the Pb isotopes were measured on average over a longer period. If a measurement showed sufficient element ratios it was labelled, and geochemistry was saved and recorded.

Samples were analysed on three different days: EK005 was completed on the 4th of November 2021; MM002, some of MM004, MM005 and MM006 were completed on the 13th of January 2022; and the final MM004 were done on the 20th of January 2022.  Results from standards are organised by day of analysis in Appendix 4.

# Data reduction

LADR software version 1.1.07 was used for data reduction of the samples. The software sets the best parameters and correction using standards across the analysis period automatically. These were examined and model formulas were corrected slightly.

Once the analyses standards and parameters were completed, each sample was examined individually and for each crystal, notes of quality and disturbance were made. Individual crystals were excluded if they had major inclusions that would alter the age analysis, showed significant evidence of Pb loss, or increased levels of common Pb that made the analyses discordant. Any analysis that was less than 75% concordant or grains that had an age with a standard error greater than 20 were also excluded. In some analysis the ablation included a rim and core age which were then trimmed and separated into two ages if possible. In other cases, the laser may have blasted into the epoxy beneath or around the zircon; these analyses were trimmed appropriately. Where analyses had to be trimmed or split the older age was typically taken. Overall, 408 laser ablation spots were performed across the samples and 76 were rejected from the analyses. It should be noted that spots 21 and onwards for sample MM004 were conducted on a different day (20th of January). The LADR/processed data can be found in Appendix 4.

# Finding the populations of the youngest zircon ages

Once the data had been exported into a formatted excel document, percentage concordance was formulated for each sample and the crystals which had less than 70% concordance were rejected along with crystals which had been noted as rejected (for reasons listed above).   207Pb corrected 206Pb/238U age were calculated for the samples that had not been rejected. After the reliable analyses had been recognized, a probability density plot was created using the add-in ISOPLOT (Ludwig, 2012) to show the zircon populations. Then, the youngest peak was used as a starting point to find the most reasonable zircon age to use to calculate the weighted mean maximum deposition age using ISOPLOT (Ludwig, 2012).  Finally, ISOPLOT was used to create Tera-Wasserberg Concordia diagrams. The age of the youngest zircon was calculated by selecting the reliable zircons in each sample and using the youngest age function in the ISOPLOT add-in.

# References

Black, L.P., Kamo, S.L., Allen, C.M., Aleinikoff, J.N., Davis, D.W., Korsch, R.J., Foudoulis, C., 2003. TEMORA 1: a new zircon standard for Phanerozoic U-Pb geochronology. Chemical Geology, 200(1-2): 155-170.

Ludwig, K. (2012). User’s manual for Isoplot version 3.75–4.15: A geochronological toolkit for Microsoft Excel. Berkley Geochronological Center Special Publication review 5: 1-75

Thompson J. M., Meffre, S. and Danyushevsky, L. 2018. Impact of laser pulse width and fluence on U-Pb dating of zircons by LA-ICPMS, Journal of Analytical Atomic Spectroscopy, 33:221-230.

Slama, J., Kosler, J., Condon, D.J., Crowley, J.L., Gerdes, A., Hanchar, J.M., Horstwood, M.S.A., Morris, G.A., Nasdala, L., Norberg, N., Schaltegger, U., Schoene, B., Tubrett, M.N., Whitehouse, M.J., 2008. Plesovice zircon - A new natural reference material for U-Pb and Hf isotopic microanalysis. Chemical Geology, 249(1-2): 1-35.

Wiedenbeck, M., Alle, P., Corfu, F., Griffin, W.L., Meier, M., Oberli, F., Vonquadt, A., Roddick, J.C., Speigel, W., 1995. 3 Natural Zircon Standards for U-Th-Pb, Lu-Hf, Trace-Element and Ree Analyses. Geostandards Newsletter, 19(1): 1-23.

Wiedenbeck, M., Hanchar, J.M., Peck, W.H., Sylvester, P., Valley, J., Whitehouse, M., Kronz, A., Morishita, Y., Nasdala, L., Fiebig, J., Franchi, I., Girard, J.P., Greenwood, R.C., Hinton, R., Kita, N., Mason, P.R.D., Norman, M., Ogasawara, M., Piccoli, R., Rhede, D., Satoh, H., Schulz-Dobrick, B., Skar, O., Spicuzza, M.J., Terada, K., Tindle, A., Togashi, S., Vennemann, T., Xie, Q., Zheng, Y.F., 2004. Further characterisation of the 91500 zircon crystal. Geostandards and Geoanalytical Research, 28(1): 9-39.