# MEMORANDUM

| TO | Tom Whiting |
| --- | --- |
| FROM | Paul Mutton |
| DATE | 23/5/2012 |
| Report No. | SGCXXXX |
| RE | Heazlewood EM modelling |

# SUMMARY

- All conductors appears to be shallow dipping to the SW at less than about 20 degrees

- There is strong current migration in the HC2-HC3-HC4-HC8 conductors to the west and the anomalies are complex indicating that the conductivity of the unit is variable, shape is irregular, and structural breaks are separating current systems.

- The conductance of the units is quite low (15S) which common for graphitic and/or pyritic units but would be very unusual (too low) for pyrrhotite/pentlandite.

- Thickness of units is difficult to determine, particularly for shallowly dipping units.

- Areas around the surface (up-dip) expressions of the conductors should be investigated and soil geochemical lines extended to check for any source of the conductors.

# METHODOLOGY

# MODELLING RESULTS

## HC1

This is a small, discrete conductor about 140m in strike and 130m in dip extent. It has a conductance of about 80S which, although not highly conductive, is the highest of any conductor in the survey area. It is about 180m deep and shallowly dipping to the West/South West. There is very little visible current migration due to its small size.

Figure 1 3D view of HC1 model with topography

## HC2-HC8

This is a conductor with complex response dues to multiple, closely spaced conductive units, and likely structural discontinuities that break up the main conductive formation that creates multiple conductors. Modelling was completed using rectangular plate models of individual line and then creating a composite model that best describes the overall shape.

The final model is shown as Plate 1. Figure 1 below is a view along strike showing the alignment of the different models which, with the exception of the two northern models, are on the sample plane indicating a single unit. It is dipping to the SW at about 20 degrees and has a low conductance of less than 20S. Typical conductors with this conductivity may be graphitic shales, or pyritic units (with/without other Cu and non conductive Pb and Zn sulphides). It is lower than the response from almost all pyrrhotite (+pentlandite) mineralisation.

Figure 2 3D view of HC2 model with topography. Most of the “separate” conductor are located in the same plane indicating that the conductor has a complex shape and conductivity distribution that is difficult to model with rectangular plates.

## HC3

This conductor is coincident with a magnetic high and initially was interpreted as possibly being a discrete conductor, however modelling indicates that is likely to be caused by a narrow, along strike extension of the HC8 conductor. It does appears to be slightly more conductive (25S), however this is still only very weakly conductive. The anomaly is superimposed on the response from HC4, which extends further to the south.

Figure 3 HC3 conductor (red) is likely to be a continuation of the HC8 conductor (black) to the south and the HC4 body (yellow) may also be associated with this unit. Looking along strike (diagram on right), most the conductors appear to be along the same horizon.

## HC4

This conductor is likely to be a parallel and below the HC3 conductor. Up dip from HC3/HC4  is a very weak conductor that may be a highly oxidised near surface expression of this conductor. This near surface position should be investigated on the ground and soil sampling may assist in prioritising the conductor.