ANCHOR MINE ENVIRONMENTAL MANAGEMENT PLAN.

Modifications

SECTION 1.0  INTRODUCTION.

Mancala Pty Ltd has acquired Spectrum Resources Australia Pty Ltd which owns the Anchor Mine, near St Helens, North-Eastern Tasmania (see Figure 1.1). The original Mineral Leases, 88M/88, 89M/88, 90M/88 and 91M/88 were consolidated into ML 55M/89.

Spectrum Resources operated the mine from 1989 to December 1991 under a Licence to Operate Scheduled Premises at a tonnage of 125,000t/a. The Licence to Operate Scheduled Premises was thereafter for a reduced tonnage of 2,000t/a. Mancala anticipates resumption of mining at a production rate in the order 125,000t/a. It also intends to use the mill to process ore delivered from other sites. The total ore milled will be less than 250,000t.

1.1  SCOPE AND OBJECTIVES OF THE EMP.

1.1.1 Scope and Objectives.

This document is based on the original EMP submitted by Spectrum Resources, which was prepared by John Miedecke and Partners in 1988. It is in support of Mancala's application for renewal of operations of Anchor Mine. It describes the proposal for reopening, the area's environment, details the possible environmental impact and the proposed project environmental controls.

The major issues....

1.1.2 Current and Required Approvals

The mine operates within Mineral Lease Number 55M/89, which is current to 1st April 2000, and Licence to Operate Scheduled Premises Number 3627, current to 6th February 1994.

The Licence to Operate Scheduled Premises needs expansion of mill throughput from 2,000t/a to 200,000t/a.

1.2   PROJECT HISTORY

Mining is believed......

The Anchor Mine.....

A number of companies....

Spectrum became interested in the property in 1986 and considered it viable as a small scale underground mining operation of the high grade lenses. It operated the mine from 1989 to the end of 1991, when failure to reach production goals and economic losses caused closure of operations. Since then the mine has been kept on care and maintenance, attended by a caretaker.

SECTION 2.0, PROJECT DESCRIPTION.

2.1     ORE RESOURCE

The tin mineralisation is hosted in an altered alkali granite beneath a generally horizontal roof contact with overlying adamellite. A tin zone of about 0.5%Sn was delineated from old workings and more recent drilling. The ore zone trends north east, with dimensions of 100 metres by 100-200 metres. The orebody consists of two lenses of thickness from four to over twenty metres. The highest grade sections of the alkali granite lie immediately below the cupola structure of the contact of the alkali granite and adamellite, which is marked by a pegmatite. These have been and will continue to be the target of the mining operation.

The remaining resource is estimated at 432,000t at 0.49%Sn, with potential for discovery of another 200,000t. This does not include the potential resource of the 300,000t of tailings, the grade of which is yet to be determined.

2.2 DEVELOPMENT SCHEDULE, PRODUCTION RATES AND GENERAL LOCATIONS

The mining plan is based on a production rate of 125,000t a year

at a grade compatible with the average resource grade, about 0.5%Sn. Most of this will be mined underground, though there is potential for production of about 20,000t from an open cut source. It is anticipated the operation will continue for five years.

The grade of the tailings will be assessed with a view to retreatment. If this proves economic, it will be done at an early stage after reopening of the operation.

The general location.......portal.

Tailings are stored in a dam close to the mine and mill. An old mine pit provides water storage.

Mining and milling is planned to be on a continuous shift basis, 24 hours a day, 7 days a week.

2.3    MINE FACILITIES

2.3.1 Mining plan and method

The mining operation will be mainly underground with a small workforce using mechanised mining methods. It is also planned to mine a limited resource open cut.

Underground mining will continue to use the established 'post pillar' mining method (see Figure 2.2), where it is deemed appropriate. A bulk mining technique will be employed where there is sufficient ore thickness and it can be done safely. This could include caving to the surface, if it can be shown to be geotechnically sound. Prior approval will be sought of the Department of Mines and the Department of the Environment and Planning, who will be advised of the details when they are formulated.

2.3.2 Mine fill, waste rock production

Where required, waste rock and coarser fraction tailings will be used for mine fill.

2.3.3 Mine drainage

The mine will..... 300 litres per minute. This will be drained to the tailings dam.

2.3.4  Mine equipment

Major mining equipment will include the following:

-twin boom pneumatic drill jumbo(s),

-5 cubic yard diesel powered LHD,

-25t diesel powered dump truck.

-20t excavator

-pneumatic airtrack drilling rig.

2.4   MILL FACILITIES

Ore processing is primarily by gravity methods.

Intended mill throughput is in the order of 25 tonnes per hour. The existing crushing, grinding and gravity circuits will be modified for improved efficiency and versatility.

A flotation circuit will be re-established to collect sulphides as a separate concentrate from the tin concentrate.

The roles of the magnetic and high tension separators will be reassessed with the view of improving the tin concentrate and recovering other marketable commodities, such as topaz.

It is anticipated the mill may be required to treat ore brought to it from other sites. The location and quantity of this ore is not known at this stage.

2.4.2  Mill equipment

The processing plant is appropriate in size to its intended capacity of twenty five tonnes per hour and remains much as it was described in the original EMP.

The mill equipment consists of:-

-a primary jaw crusher,

-a secondary jaw crusher,

-a Barmac impact type tertiary crusher,

-a small rod mill,

-a two cell jig,

-six tables,

-eight spirals.

A flotation circuit will be added.

The equipment in the drying shed comprises:-

-two diesel fuelled heaters,

-a high tension separator,

-a magnetic separator,

-concentrate bagging equipment,

-a forklift.

2.4.3   Processing chemicals

The only chemicals......collector.

Xanthate and the auxiliary collector are substances of low toxicity, which are commonly used in flotation processes. The amount used relates to gravity concentrate rather than ore milled. Therefore the annual use of each is in the order of 70

kg.

A small lime addition may be necessary to raise the pH of the flotation circuit above 8. This is unlikely to exceed 0.5kg/tonne, if required, amounting to an annual usage up to 350kg.

2.4.4   Tailings disposal

The tailings flow to a storage dam located close to the mill.

Water is recycled from the tailings dam as required.

All aspects......in Section 4.

2.5   TRANSPORTATION

2.5.1 Access

The site is accessed via the gravel Anchor Road which turns off the from the Tasman Highway about 17km east of St Helens. The mine is about five kilometres north of the Highway.

A short access road was constructed to reach the mine from the Anchor Road.

The Anchor-Lottah road  to Lehner's Ridge is not expected to be used in its current form. It would require upgrade before it could be used routinely by trucks.

2.5.2  Transport

Vehicle movement is  generally light: normally up to 12 a day. Deliveries of mining and milling machinery will occur during the reopening stage. Thereafter, the fortnightly trucking of 20 tonne lots of tin concentrate would be the only routine heavy vehicle use of the access road. There will on occasions be delivery of ore from other sites, in which case there would be a number of trucks using the road in a short time. Heavy vehicle traffic will be confined to the hours of 7 am to 6 pm.

2.6   INFRASTRUCTURE

2.6.1  Water supply

The water containment measures outlined in the first Environmental Plan were implemented by Spectrum Resources. Anchor Creek was diverted by a cutting round the north west side of the old open cut to join Ariel Creek (see Figure 4.1). The combined flow is dammed in a gloryhole excavation in the western corner of the old open cut.

The water level in this dam was raised 4.9m by the building of an earth dam wall across the gully in which an adit emerged from the glory hole. This provides approximately 18,000 cubic metres of storage. The overflow of the gloryhole dam runs down a spillway cutting, which diverts the flow west of the old battery and thence to the Groom River.

This provides a back-up for the plant water supply, which is mainly taken from the sediment dam below the main tailings dam (see figure 4.1). This arrangement is sufficient for the plant's water requirement.

2.6.1   Power supply

A Hydro Electric Commission powerline passes over the valley, above the tailings dam. The mine's power supply is taken from this powerline.

The site power demand is about 375KW.

2.7   WORKFORCE AND HOUSING

The mine's permanent workforce will number about 17 people, none of whom will live on site.

Office, changeroom, shower and toilet facilities are provided in four prefabricated, truck-delivered units, which are arranged in a square on the south side of the main access track (see Figure 2.1). Sewage and domestic waste water are contained by a septic system.

While the mine remains on care and maintenance a caretaker lives on site.

SECTION 3.0   PROJECT AREA ENVIRONMENT

3.1   INTRODUCTION

A number of environmental surveys were conducted over the area in preparation of the original EMP by John Miedecke and Partners Pty Ltd. These included:

-the nature conservation values,

-archaeology,

-hydrology,

-water quality,

-soils and rehabilitation potential and

-aquatic fauna.

The essential ........below.

3.2  GENERAL DESCRIPTION

The mine .....about seventy five years ago.....forest.

The site of the current mine development has been consciously kept within the old open cut workings of the Anchor Mine (L. Newnham, pers comm.), and all the surface facilities are restricted to this area.

The Groom River is the main waterway, traversing the lease south of the mine site.

3.3   NATURE CONSERVATION VALUES

The vegetation and flora of the lease area were studied by J.B. Kirkpatrick and P.J. Cullen (on behalf of John Miedecke and Partners), who mapped the plant communities and established their conservation significance. Their report is Appendix A.

3.3.1   Plant communities

(as is to end of 3.5.2)

3.6   ARCHAEOLOGY

Mining commenced......1.75 million tonnes of ore.

Historical records and photographs exist in the St Helens History Room. The proprietor, Mr Peter Burns, is very willing to discuss the history of Anchor Mine and to provide copies of photographs and written material for anyone interested in it.

The remains of the old battery, a local tourist attraction, is located near the outlet of the water storage and anfo mixing shed (see Figure 2.1). The remaining equipment consists of two braces of stamp batteries in basically good condition, except for some vandalism and rotting of the timber bases, which will eventually cause the batteries to fall over. The batteries are of two makes. The eastern brace of two five head stamps was made by I.E.E. Salisbury Engineer of Launceston, and are dated 1882 and 1883. The western brace, also of two five head units, was made by C. and D. Thompson and Co. Makers of Castlemaine, Victoria. These were brought to this site in the 1930s (pers comm C. Bacon 5.7.1993), possibly from the original Anchor battery.

Two Wifley tables and a puddle jig, in poor condition, remain in the gully a few metres down from the batteries, among the manferns. The bin that used to feed the batteries is entirely hidden in a bank of soil which is so well grown over that it is indistinguishable from natural forest floor. The encroachment is right to the back of the batteries, partly obscuring drive wheels and shafts between and behind the stamps. It is not known if the primary crusher is still in position behind the feed bin.

This machinery was presumably driven by a Pelton wheel, which was the much smaller and more efficient successor to the massive water wheel. It was run by the diverted Ariel Creek.

Of the original ambitious hundred stamp battery, which was situated on the eastern side of the valley, there is no sign; nor of the 66 foot diameter water wheel, which carried 10 tons of water at any given time, that drove it when there was a sufficient supply of water. It was replaced by a series of Pelton wheels in 1913.

3.7    SOILS AND REVEGETATION

This section remains unchanged from the original EMP by John Miedecke and Partners Pty Ltd.

3.7.1 Soil survey

(as is)

3.7.2  Soil types

(as is)

3.7.3  Rehabilitation suitability

(as is)

3.8   LAND USE, ZONING AND TENURE

The Leases are on Crown Land, designated as State Forest and zoned Public Purposes.

There is no residence close to the mine. The remnants of the township of Lottah is a few houses over a kilometre away. Two

residences close to the Anchor Road were noted in the original EMP. In the interim, another residence has been established about 2km down the road south of Anchor Mine. These rely on tank and dam water, rather than take their supply from the Groom River.

SECTION 4.0    PROJECT ENVIRONMENTAL MANAGEMENT

4.1   INTRODUCTION

The Anchor Mine is set.....

It is anticipated the known resource of nearly 450,000t will expand with exploration during the mining operation to about three quarters of a million tonnes. Therefore, this particular phase of mining should last about six years.

The proximity of the Groom River, which is the water supply of St Helens, necessitates containment of all mine and process water to the site. The environmental management plan is intended

to ensure all environmental effects are minimised.

The intended measures for tailings storage, water management and rehabilitation are discussed in this and following sections of the EMP.

Though the valley has suffered much disturbance from past mining activity, there is remarkably little of it that has not been grown over and reclaimed by the rainforest. The current operation is confined to a relatively small area of the old workings. The mill and workshop are sited in quarry hollows of the old open cut. The tailings dam, obviously the most prominent feature of the operation, is also sited within the area of the old open cut.

Of the old workings, the signs are rock faces and, amongst the trees of the hillsides, mine benches, well grown over. The old stamp battery, tables and jig, and occasional pieces of pipe are the only mechanical remains of former operations. The battery has appreciable historical value. The new operation will be conducted in the spirit of preservation of artefacts already established by Spectrum Resources.

The three main objectives of the Environmental Management Plan, therefore remain those stated in the original plan:

-to minimise the effects of the operation on the rivers and       streams of the area.

-to minimise the direct impact of the operation on the           vegetation and to rehabilitate unavoidable damage.

-to preserve the historic value of the site.

The operation is required to comply with the EPA standards for emissions into inland waters.

4.2    IMPACTS OF THE OPERATION

In the original environmental study, John Miedecke and Partners Pty Ltd established a priority list of potential environmental impacts of the operation, as follows:

-modification of the habitat....

-the effect on the vegetation communities ...

-disturbance....

-other more....

Basically, these have not changed, except the construction phase is five years past, and the tailings dam has three years production (about 300,000t) in it.

4.2.1   Aquatic Habitat

Spectrum Resources went to great lengths to ensure that no mine and process water escaped from the site, though its environmental consultant (Miedecke 1988) confirmed by test work that the risk of serious contamination is low. Miedecke indicated that fluorine level in drainage water was the only potential contamination problem. This is a function of the fluorite content of the greisen host rock, which mainly stays in its crystal form in the tailings, though evidently there is minor dissolution to cause the slight level of fluorine in the tailings leachate (see 4.3.1 below), probably from the finer ground fluorite. Previously monitored discharges from the site comply with the standards for emissions into inland waters set by the Department of Environment and Planning and will continue to do so with the reopened operation. Regular monitoring will confirm this.

Containment of sediment carrying waters from the operation is described in Section 4.6.

4.2.2   Direct Impacts on Flora and Fauna

The construction phase of the operation is far enough in the past for the site to be a familiarity for the fauna of the area, particularly because of its relative inactivity, and there are signs of this in spoor and droppings, despite the presence of the caretaker and his two dogs. No doubt the noise of the resumption of operations will discourage larger animals coming close until they become used to it.

Reclamation growth by trees and shrubs has been rapid on all areas that are not bare rock, tailings or made road, including on the tailings dam starter embankment and the sediment dam wall. The high rainfall of the area (quite comparable to the West Coast at two to three metres per annum) has no doubt greatly contributed to this fecundity. In fact, apart from the presence of the current operation, it is hard to see the hollow was once laid bare by the old workings.

4.2.3  The Old Battery

The old battery, described above, has not been affected by the current operations. Contact has already been made with the St Helens History Room proprietor, Mr Peter Burns, opening discussions for cooperation in preserving this artefact.

4.2.4   Other Impacts

The other environmental impacts of the operation include noise, dust and visual, but because of its remoteness, climate and isolation from residences, these are not considered significant public inconveniences. It is possible the three residences on the Anchor Road might receive some dust on particularly dry days with northerly winds.

4.3   WASTE CHARACTERISTICS

4.3.1  Geochemical Characteristics of the Tailings, Ore and            Waste Rock.

Before reopening the mine, Spectrum Resources arranged geochemical and leaching tests of the ore and tailings.

Samples of  ore......(no real changes through to and including p27, except for the changes in pencil).

4.4.4   Stream Diversions and Runoff Control

Anchor Creek was diverted to the water storage (see Figure 4.1) before the start of operations by Spectrum Resources.

Provision was also made to control precipitation and runoff into the tailings pond and to ensure the dam embankment is not overtopped.

A tailings pond is maintained at the rear of the dam area for recirculation of plant water and to assist in precipitation of ultra fines. An emergency spillway to discharge surplus water in winter and during rainstorms was cut through a narrow rock abutment at 246m RL on the western side of the tailings dam to discharge to the water storage pond. This spillway will be progressively raised as the level of the tailings dam rises. At higher levels a new channel will be cut in the bank further to the rear of the dam to form a permanent discharge on abandonment.

4.4.5   Embankment Construction

The tailings dam wall is of rock construction with its base at 238m RL and crest at 246.5m RL, and made of about 10,000 cubic metres of rock fill. This is underlain by a trench excavated across the creek bed, which was filled with rock, to ensure that no fines layers would form beneath the dam structure.

A rock drainage filter was incorporated into the structure to ensure continuation of drainage under the dam wall.

The starter dam was a simple structure designed to impound tailings and is intended to remain permeable to ensure good drainage during operations and after abandonment.

The dam has already been raised above the 246.5m RL by placement of coarse sand tailings in successive small lifts. This was deposited by gravity pipeline. Pumping will probably be required as the gradient decreases.

Spectrum Resources submitted plans, specifications of the dam and details of stability factors for consideration and approval by the Department of Mines.

4.5    OTHER WASTES AND CHEMICALS

4.5.1  Other Wastes

Sullage and septic tank drainage are directed to the tailings dam in accordance with Local Government Regulations.

Flammable refuse......

4.5.2   Fuels and Chemicals

No potentially hazardous chemicals are used nor created during processing.

Storage of diesel fuel and oils is in a bunded segregated area. Estimated fuel consumption is still in the order of 8,000 to 10,000 litres per year.

Oil and grease consumption is between 800 and 1,000 litres per year. These are stored in 50 litre drums in the bunded area.

Drip trays are set beneath fuel and oil outlets.

Flotation chemicals are stored in drums in the mill building or in an open area, as appropriate.

4.6   WATER MANAGEMENT

4.6.1  Water Balance

The rainfall/evaporation balance for the area indicates that a closed water circuit can be maintained in summer only, because the rainfall is high during the rest of the year, while the evaporation rate is low. Figure 4.2 shows the rainfall /evaporation balance for the nearest stations to the site. However, the mine's location, in a hollow that receives only a few hours sunlight a day, at best, means the rainfall is probably higher and the evaporation is lower than at the recording stations of Ringarooma and Scottsdale.

Therefore, it is inevitable there is excess site water run-off from April to October.

FIGURE 4.2

4.6.2  Water Management System

The purpose of the water management system is to ensure a continuous supply of water to mining and milling operations, and to manage on-site water in a manner that any discharges comply with the environmental regulations, needing minimal treatment.

Figure 4.3 shows the water management system.

The various contaminated water streams have been identified above (quantities and qualities), and options for any reuse, treatment or release have been considered. On-site water use has been maximised by containment and recirculation of contaminated water.

A major purpose of the system is to prevent pollution of the Groom River.

4.6.3   Water Supply

Plant water supply is mainly from the sediment dam, augmented by free water on the tailings dam (remembering that the mill discharge is basically water carrying only rock fines, it being primarily a gravity plant). Top up supply, in times of recycled water shortage, is from the water storage pond  in the diverted Ariel and Anchor Creeks (W3 in Figure 3.2 and `water storage' in Figure 4.1). In the recent operations this system has proved more than adequate in supplying the requirements of the site.

4.6.4   Mine Water and Contaminated Water From Ore Stockpiles           and Mill Spills

4.6.4.1  Mine Water

The various sources of water contribution to the mine discharge are:

-natural mine inflow of ground water,

-water used for drilling,

-water delivering hydraulic mine fill from the mill.

As anticipated in the original EMP, the mine water inflow is relatively minor, and to date, no problem to the operation. Fine mud and puddles occur in the underground workings, but the outflow is minimal. The ground water quality is low in salinity and neutral in pH, with the solids it carries being the only problem. This is dealt with by channelling it to the tailings dam where the solids settle out.

Water used for drilling is obtained from the water storage tanks on the hill above the mine, to which water is pumped from the storage reservoir. High quality water is required for use in the rock drilling machinery to avoid mechanical damage. The operation of a jumbo drifter requires approximately 60 litres per minute which joins the natural inflow as part of the mine outflow.

To date there has been no filling of stopes with hydraulically delivered mill waste. There is still a possibility the cut and fill mining method will be used, in which case hydraulic fill will be required. The water component of the fill would amount to about 300 litres per minute. Most of this water drains off, leaving some pore water in the fill. The drained water will form part of the mine effluent. It is likely the fill will be cemented to give it some cohesion. The resulting drainage water will have a high pH (about 12) which will have a positive effect of helping the flocculation and settlement of fines in the tailings.

The combined active mine effluent will peak at times, with washing down of faces and muckpiles as well as that used in drills, at 250 to 300 litres per minute.

The mine water carries some solids in the form of mud at present. This might later be augmented by water from hydraulic mine fill of tailings, should the mining method adopted call for it. There is a slight potential for elevated levels of fluorine in this outflow, as in the tailings (see 4.3.1 above). All mine drainage is directed to the tailings dam as part of the water management system.

4.6.4.2 Ore Stockpile and Mill Area Drainage

The drainage from ore stockpiles is muddy and likely to contain low levels of fluorine, consistent with that of all the rock broken in the mine. This, with all the general drainage from the site, is also directed to the tailings dam.

4.6.5  Tailings Storage and Discharge

Diversions are in place around the tailings dam to minimise inflow of meteoric water, to maintain maximum availability of space for tailings, mine, stockpile and site drainage.

The original design was supposed to raise the effective height of the tailings dam wall by cycloning the tailings and piping the coarse fraction to the front of the dam. The fine fraction was to be piped to the pond thus formed behind the dam wall. This, as described in the original EMP, is normal practice and will be continued by the new operators. However, it must be noted at this point that the front slope of the wall was built not just with the coarse fraction as originally planned, but with the whole of the tailings flow, coarse and fine fractions, and made much steeper than specified in the original design (pers comm L. Newnham, June 1993 and M. Stevens, July 1993).

The design intended the area of the tailings pond should be kept as large and as shallow as possible to ensure optimum conditions for precipitation of fine sediment. Liming was also to be used to facilitate precipitation.

Some mill process water is reclaimed from the tailings pond.

The design of the tailings embankment should allow water to pass through the embankment to flow into the sediment dam (see Figure 4.3). However, again it should be here noted, the practice of building the dam up with unsorted tailings has resulted in water retention in the dam wall. This makes it unstable and slowly mobile. Since cessation of operations in late 1991 the tailings wall has been observed to have moved forward about 0.3m (pers comm M. Stevens, 16th July 1993).

The water which flows through the tailings dam wall (though to a lesser extent than was envisaged in the original design and incidently carrying fines with it) flows into a sediment dam with a relatively impermeable earth wall (see Figure 4.3). Excess water from this dam flows down a spillway into a marshy area which provides wetland treatment of the tailings effluent before it finally flows into the Groom River. This system has proved successful to date in preventing any sediment of the operation reaching the Groom River.

4.6.6  Discharges From Site

At worst, the water quality in the tailings storage is equivalent to that depicted in Table 2 (Section 4.3.1). This complies with EPA emission standards. Fluorine is the only element of concern in tailings leachate, for its level may become slightly elevated as it passes through the tailings dam. The saturation extracts in Table 2 represent concentrations of water in the milling process sufficient to achieve saturation in the samples. Much more water than this is used in the milling process and therefore the actual concentrations are much lower. The amount of elements dissolved in the tailings effluent depends on the amount leaching from the tailings compared with the throughput from the surface.

This water is recycled in dry periods when concentrations tend to be highest.

Water quality standards apply at the discharge from the final treatment area, prior to release to the Groom River.

4.7   DISTURBANCE AND REHABILITATION

4.7.1  Current Situation

The site is long past the construction phase anticipated in the original EMP, and is well established. The disturbed ground is now growing seedlings of the native flora, with many trees and shrubs over a metre high. Significantly, this includes the walls of water storage, tailings and sediment dams, helping to bind them together. The tailings dam starter embankment is of rock, while the walls of the other two are of earth.

The absence of noise during the mine's inactivity has evidently encouraged return of much of the fauna.

Miedecke's assessment of the potential for revegetation of the site is well vindicated by the evidently strong regrowth around the site.

There is, however, one legacy of the previous operators which requires early attention, being a small dump of sulphide concentrate floated off in the process of improving the quality of the tin concentrate. This is located near  the northwestern edge of the tailings dam, close to the mine rubbish tip, which is a legacy requiring less immediate attention.

4.7.2  Soil Salvage and Conservation

In the development phase of the site the topsoil was stockpiled for use in subsequent rehabilitation.

4.7.3  Rehabilitation

The programme of rehabilitation outlined in the original EMP stands basically without change.

Most effort will be required on the tailings dam because of its three hectare area and greatest difference from its state prior to the current operation. Its final form will be close to that envisaged by Miedecke, as shown in section in Figure 4.4.

The plan to progressively rehabilitate the face of the embankment as it was raised by putting soil on it was not implemented by Spectrum Resources in its operations from 1988 to 1991. The new operator is more likely to do this in one operation at closure of the mine, than piecemeal in steps. The decision on this will be made when the new operators are more familiar with the problem posed by the building of the tailings wall in a manner which differed from the design method.

It is understood, as a result of inspection of the site with a former manager, L. Newnham, on the 6th June 1993, that the build up of the dam wall with the coarse and fine fractions of tailings by the former operators, prior to the December 1991 closure, was much steeper than planned and described in the original EMP. This greatly changes the permeability and stability of the front slope, and the measures required to protect it from erosion need rethinking as a result. A likely solution will be to lower the slope angle with an excavator, forming berms at 5m vertical intervals, as originally proposed, then covering the slope with topsoil, which will be seeded and spread with fertiliser.

The embankment will require protection from browsing animals until the vegetation is well established. The seed mix proposed by Miedecke (see Appendix E) will be used for initial treatment of the topsoil. Follow up seeding will be done the next autumn if required. However, the evident rapidity of regrowth at the site makes it unlikely a second treatment will be required. The natural distribution and germination of seeds is evidently very effective in this locality.

The surface of the dam, an area of about 3ha, will also be covered with soil and seeded. Natural seeding will likewise need little encouragement there.

Disposal of the cache of sulphide concentrate will probably be done by putting it through the plant again to float it free from contaminants picked up in digging it out of its current repository and selling it.

4.7.7  Fire Control

The new operators will maintain strict control of fire and enforce fire bans in dry weather, in the continued interest of conservation of the area.

4.7.8  Feral Animals and Diseases

The policy of preventing introduction of undesirable plants, animals and diseases to the property will be continued by the new operators.

4.8  OTHER ISSUES

Dust and noise of the mining operation are insignificant in effect on the residences closest to the mine because of their distance away and the interposition of wooded hills.

If the open cut is reopened, there will be more noise and dust. The noise will probably be audible at the closer of the residences, but it is unlikely to be more intrusive than the noise of traffic in a small town. The duration of the open cut operation is unlikely to be for more than a few months.

The reopening of the mine will mean there will be more than the usual traffic passing the  residences on the Anchor road, though this will still be only of a small volume.

4.9  MONITORING

It was intended from the outset that the reopened Anchor Mine operation meets designated established environmental standards. The new operators will comply with these standards and continue the monitoring programme established by Spectrum Resources.

The results of the monitoring will be forwarded to the Department of Environment and Planning on a regular basis.

The operation is not currently under any ministerial exemption.

Rehabilitation progress will also be monitored.

4.10  DECOMMISSIONING

At cessation of operations, the site will be cleared of all plant  and buildings, except for that of historic value, eg, the old stamp battery described above in Section 3.6.

The proposed rehabilitation of the tailings dam is described in Section 4.7.3, above. It is considered prudent to retain the seepage collection dam and wetland marsh intact to continue their function, as benign aquatic habitats in the forest, which will certainly encroach the site in time, much as it had over the old workings prior to reopening in 1988.

The mine portals will be blocked with waste rock as large as obtainable to prevent casual access to the mine.

The site rubbish tip will be cleared and removed to the nearest municipal disposal site. It does not, to our knowledge, contain any toxic or hazardous material.

Prior to abandonment, the Department of Environment and Planning will be contacted to arrange a site inspection.