BLUE TIER TIN MINES: PRODUCTION AND SAMPLED GRADES.

INTRODUCTION

I visited some of the abandoned Blue Tier tin mines on the 31st August 1993, guided by Peter Burns of the St Helens History Room. We looked at the Mt Michael, Summit, Australia, Don and Liberator Mines, all open cuts. Each mine was extensively overgrown, with the Mt Michael and Summit being the more exposed because of their positions on the top of Blue Tier.

ACCESS

The Mt Michael Mine is reached via a track turning north off the Sun Flats Road, about 1.5km north of the site of Poimena village, on top of Blue Tier. This track was originally properly formed and is still negotiable by two wheel drive vehicle with high clearance. The mine is 1.5km from Sun Flats Road.

The Moon Mine is visible and easily accessible from the Sun Flats Road, to the east of the road about 1km north of Poimena.

The Summit Mine is about 1km south-west of Poimena. A track put in by an exploration company in the 1960s is defined by deep ruts, but as it was not built up is now attemptable only by the foolhardy and those fond of being bogged. We parked the car at Poimena and walked an old pack track which joined the exploration track, now heading south.

We continued past the Summit Mine and followed the bed of a former self-acting tramway, now thoroughly overgrown, south down a steep hill about half a kilometre to the "Puzzle", subsequently known as the Australia Mine.

The Don Mine, further down hill from the Australia, was reached by walking north through the bush from about 1.3km along the Lottah Road west of the turnoff to Anchor Mine for about 200m, crossing Crystal Creek.

The 400m walk to Liberator Mine starts about 2.3km west of the Anchor turnoff, where a well defined walking track leads south from the road.

SIMPLE GEOLOGY

All the mines are in greisen type deposits formed at the top of the alkali feldspar Lottah Granite, which intruded the biotite adamellite Poimena Pluton, very much like the Anchor deposit. Therefore the orebodies are low grade nearly horizontal deposits of cassiterite in the darker zones of altered greisen of variable thickness.

In general the grades were below 1% Sn, though richer pockets of over 2% Sn were encountered.

The rocks encountered in this excursion were almost all the biotite adamellite of the hangingwall. The greisen was positively recognised only in some fragments beside the Liberator pit.

PRODUCTION AND SAMPLE GRADES

Mine        	Production t.  	Grade %Sn   	Sample length m.  	Grade%Sn

Crystal Hill     	-             		-          		10          	0.37

"            	-            		 -               	17          	0.44

Liberator     1,600          		 0.69         	 ?          	0.40

North Liberator  -             		 -               	?        		0.2-0.25

The Don       4,054           		0.88            	30.8         	0.47

"            	?            0.5

" early      	?           1.5-2.0

Australia        ?             2.27

"            	?             		0.66          	 60.96        	0.50

"            	?             0.40

" upper bench                               			21.3         	0.77

" lower bench                               			21           	0.54

Summit     no records

Victory    no records: veins

Moon       no records                        16m in DDH    0.60

"                                          		1m in DDH     1.35

"                                          		1m in DDH     0.85

Mt Michael  no records                       DDHs          nil

G.I. 1.9.93