FILE NOTE

SUBJECT:	Transformation of Bold Head drill hole database from Bold Head Mine Grid (BHMG) to GDA94 coordinates, Feb 2018 – King Island Scheelite (KIS)

DATE:	21 Feb 2018

Tim Callaghan was contacted to provide a Bold Head drilling database with collars in AMG or GDA coordinates.  The database was provided but data was only in local mine grid.  KIS do not have an accurate transformation to convert BHMG coordinates and all their work with historic data has been in mine grid.  Their only method to convert coordinates is based on approximate pick-ups of two historic drill holes (collar positions estimated on old pads) and the inferred location of the vent rise.

The three points, in three grid coordinates were provided but it was stressed the locations can now only be located roughly on the ground, so the ISG and GDA coordinates are only approximate.

| BOLD HEAD GRID TRANSFORMATION (Accuracy 5m) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Pt | Y_BHG | X_BHG | Y_ISG | X_ISG | Y_GDA | X_GDA |
| BH360 | 10250.95 | 40281.12 | 566845 | 220246 | 5564828.21 | 249722.68 |
| BH361 | 10659.21 | 40194.88 | 567200 | 220041 | 5565178.65 | 249509.66 |
| Vent Rise 1 | 10517 | 40465 |  |  | 5565147 | 249808 |

A search of Tasxplor reports shows that no record seems to exist of the relationship of BHMG to ISG, AMG or GDA.   However, a search of the mine plan map catalogue found that MRT does have an old Geopeko mine plan (0374_1562) which shows several drill holes plotted on three grids, including BHMG and ISG (plus an earlier imperial mine grid).

This plan was registered in Mapinfo using the ISG grid.

Four marked points in BHMG were then measured from the plan and ISG coordinates read off in Mapinfo.

BHMG				ISG

10500N 40450E			567140.6N 220327.4E

10650N 40450E			567282.8N 220280E

10500N 40300E			567092.5N 220184.6E

10650N 40300E			567234.9N 220137.3E

Mapinfo was then used to convert ISG to GDA94 coordinates

BHMG				GDA94

10500N 40450E			5565125.56N 24979.46E

10650N 40450E			5565266.71N 249746.85E

10500N 40300E			5565074.24N 249655.72E

10650N 40300E			5565215.59N 249605.21E

An affine grid transformation was then calculated in Mapinfo to relate BHMG to GDA94:

"Bold Head Grid", 3008, 116,7, 147, 0, 0.9996, 500000, 10000000, 7, 0.93735899881, 0.335524946451, -2060938.3125, -0.33990480678, 0.940279951319, -5137368.609375, -500000, -500000, 10000000, 10000000

This was then used to convert the Bold Head drilling database collars to GDA94 coordinates.

The angular relationship between BHMG and ISG was measured from plan 0374_1562 as 18.5° east.  GDA94 is 1° east of ISG so an angle of 19.5° was subtracted from the BHMG bearings.

As a check, plan 0374_343, which is a 1:500 surface plan of the portal area in ISG coordinates, shows the location of hole BH358 which can be measured accurately as ISG 566670N, 220122E. When this is converted to GDA94 it gives 5564650.3N 249602.6E.  The Mapinfo grid transformation from BHMG to GDA94 gives 5564650N 249600E, so within 2.5m.

I contacted Tim Callaghan and asked him if he could see any problems with the transformation and his comment was that he wasn’t aware of the Geopeko plan and the transformation should be more accurate than what they’ve been using to date.

I’m happy the calculated GDA94 collar locations are accurate to within 5 metres.

SR 21-2-2018