Mineralogical/Petrology Report

LJN2018-143

# Mineralogical Analyses, 
Thomas Ck Prospect

An unpublished Mineral Resources Tasmania Report for:

R Bottrill

By:      	R S Bottrill

Date:   	29/07/2019

By:      	R S Bottrill

Date:   	29/07/2019

By:      	R S Bottrill

Date:   	29/07/2019

By:      	R.S. Bottrill and L Unwin

Date:   29 July 2019

By:      	R S Bottrill

Date:   	29/07/2019

By:      	R S Bottrill

Date:   	29/07/2019

# Summary

# The vein contains pink albite with green amphibole and quartz.

# Introduction

I inspected a drillhole and tested one mineral sample from the Thomas Creek prospect (Table 1), to determine its mineralogy.

Table 1: sample details.

| Reg. No | Location | Description |
| --- | --- | --- |
| G409138 | Thomas Ck TCDD004/226.5 | albite-actinolite? |

# Sample descriptions

Under the stereomicroscope the sample contains mostly mid-green weakly fibrous amphibole, with a pink mineral (feldspar?).

# Fig. 1: Green amphibole and pink feldspar from a vein. Sample G409138. FOV: about 90 mm.

# XRD

The samples were prepared, examined and analysed in the MRT laboratories, Rosny Park, Tasmania. They were run on a Rigaku Miniflex 600 X-Ray Diffractometer system: a 600W generator 150mm goniometer with a Cu tube; 40kV/15mA, sample spinner and a D/teX Ultra High Speed 1D Detector with Be window, -3o to 145o 2Ø scanning range and 2o - 140o 2Ø measuring range, with a scanning speed of 0.01 to 100o/min, A graphite monochromator and a Kß Ni- filter, The analysis software used is the PDXL2 using the ICCD database.

# XRD Results

# The results are shown in Appendix 1, and indicate the green parts of the sample are rich in amphibole (ferro-actinolite?) and chlorite (corrensite?). The pink material is albite, calcite, maghemite and quartz.

# R.S. Bottrill 						L Unwin

# Mineralogist/Petrologist 			Technical Officer

Disclaimers

While every care has been taken in the preparation of this report, no warranty is given as to the correctness of the information and no liability is accepted for any statement or opinion or for any error or omission. No reader should act or fail to act on the basis of any material contained herein. Readers should consult professional advisers. As a result the Crown in Right of the State of Tasmania and its employees, contractors and agents expressly disclaim all and any liability (including all liability from or attributable to any negligent or wrongful act or omission) to any persons whatsoever in respect of anything done or omitted to be done by any such person in reliance whether in whole or in part upon any of the material in this report. The MRT laboratories are not NARA registers but work to similar standards.

# This and other data collected in MRT laboratories may enter the MRT databases but every attempt will be made to ensure it remains closed file and not be available externally, unless at your request.	
 Appendix 1:  XRD Laboratory Report

Client:  R. Bottrill

Sample Source:  Thomas Ck

MRT Job Number:  LJN2018-143

Analysis:  Approximate Mineralogy

Method:  X-Ray Diffraction

Results:

Analysis Results - G409138

General information

| Analysis date | 2018/12/19 15:39:29 |  |  |
| --- | --- | --- | --- |
| Sample name | LJN2018-143-Thomas Ck | Measurement date | 2018/12/05 15:38:20 |
| File name | G409138.ras | Operator | lunwin |
| Comment | Shifted -0.05 |  |  |

Quantitative analysis results

| Phase name | Content(%) | Formula |
| --- | --- | --- |
| Ferro-actinolite | 44(7) | Ca2 Mn0.2 Mg1.9 Fe2.7 Fe0.3 Si8 O22 (OH)2 |
| Quartz | 34(5) | SiO2 |
| Maghemite | 9(13) | Fe2O3 |
| Albite | 9(3) | NaAlSi3O8 |
| Calcite | 2.3(7) | Ca (CO3 ) |
| Corrensite | 1.0(3) | (Mg, Al)9 (Si, Al)8 O20 (OH)10 ·4H2O |

Analyst:  L Unwin

Date:  22 December 2018