MobileMT Electromagnetic Technology

Mobile MagnetoTellurics (MobileMT) is the latest innovation in airborne electromagnetics and the most advanced generation of airborne AFMAG technologies.  This newly developed approach to airborne electromagnetic data measures three orthogonal magnetic field components in the air and two electric filed components in the ground base stations (impedance-type data).  The system is designed and implemented in order to overcome existing limitations of airborne techniques based on passive electromagnetic fields principles and, ultimately, for improving exploration efficiency.

MobileMT is a passive airborne electromagnetic technique that records magnetic (in the air) and electric (on the ground) fields generated by natural sources in the audio frequency range.  The natural electromagnetic primary field sources for MobileMT are considered with frequencies ranging from 25 Hz to 30 kHz (ELF+VLF) which provides considerable depth of investigation from the surface to >1 km.  The exploration system includes two pairs of grounded electric wire lines, one of them is for reference signal, and moving three-component inductive coil system softly suspended and with low-noise signal amplifiers for magnetic field measurements (dB/dt) in three orthogonal directions.  A crucial element of the technology is the capability of aerial acquisition magnetotelluric data in four decades frequency band.  The auxiliary equipment includes Geometrics cesium magnetometer G-822A, GPS navigation system, radar altimeter, data acquisition system.

Mt Read Project MobileMT

During mid-January 2019 Expert Geophysics Limited (EGL) flew a MobileMT survey for Accelerate Resources Limited over specific prospect areas (Thomas Creek, Mt Lowren and Timbertops) within the Mt Read Project located on the Sorrell Peninsular in western Tasmania.  A total of 430kms of MobileMT survey data were acquired over an area of approximately 104sqkms.  All surveying was flown at between 200-400m line spacing along north-south lines with some limited cross/tie lines coverage over the central main Thomas Creek prospect area to provide additional detail.

The purpose of the survey was mapping bedrock structure and lithology, including possible alteration and sulphide mineralization zones, using apparent conductivity corresponded to different depth levels and the magnetic properties of the bedrock units.  All resultant final MobileMT data provided by EGL was inverted via Computational Geoscience Inc. to generate 3D conductivity results mapping both high and low discrete conductive zones.

Preliminary evaluation of the resultant 3D inversion results has defined a number of extensional target zones for exploration follow-up at each of the three main prospect areas plus also more regional type targets (see Figure 1 below).  Primary target conductive zones are marked by bold yellow outlines with initial notable ones being present on the NE side of the Thomas Creek prospect, the north side of the Timbertop magnetic target and at Mt Lowren central relating to the local magnetic body/ultramafic unit - these may relate directly to sulphide mineralisation.  Many other regional targets are present plus also clearly conductive near surface zones relating to either topographic lows/drainage patterns or the coastal margin in the far SW of the survey coverage.  Ground follow-up/site checks are required for all primary targets and then potentially the more regional type targets distilled from full/final interpretation.  Several high resistivity (very low conductivity) bodies were also identified which may highlight the presence of alteration, notably the one present central to the Thomas Creek circular magnetic complex.  Figure 2, 3 and 4 below provide a zoomed in view of the three main prospect areas/MobileMT results - Thomas Creek, Timbertop and Mt Lowren respectively:

Figure 1 – MobileMT 3D Inversion Conductivity Isosurfaces (Conductive to Resistive - red to dark blue), primary targets yellow bold outline over various relevant imagery (224Hz AppCond, DEM and RTP Magnetics)

Figure 2 – MobileMT 3D 3D Inversion Conductivity Isosurfaces (Conductive to Resistive - red to dark blue), primary targets yellow bold outline over various relevant imagery (224Hz AppCond, DEM and RTP Magnetics – left to right) – Thomas Creek

Figure 3 – MobileMT 3D 3D Inversion Conductivity Isosurfaces (Conductive red high to low light blue), primary targets yellow bold outline over various relevant imagery (224Hz AppCond, DEM and RTP Magnetics – left to right) – Timbertop

Figure 4 – MobileMT 3D 3D Inversion Conductivity Isosurfaces (Conductive red high to lower orange), primary targets yellow bold outline over various relevant imagery (224Hz AppCond, DEM and RTP Magnetics – left to right) – Mt Lowren